Chapter 3   Basic Techniques

On the IE3D, a circuit is described as a set of polygons, and a polygon is described as a set of vertices. This chapter will explain the basic editing skills by going through the construction and analysis procedure of a chamfered bend step by step.

Before you run MGRID, it is suggested that you make notes to your circuit. First you should introduce the x and y-coordinate system to your circuit. The x and y-coordinates of each vertex should be marked. Distances between vertices should be calculated if necessary. Try to break a circuit into several pieces when a complicated circuit is encountered. Try to find a minimum length such that other lengths are multiple of this minimum length. This minimum length can be used as the grid size for mouse input. The above preparation is beneficial to the circuit construction process.

# Section 1. Length Unit, Layout Parameters and Meshing Parameters

We are going to construct the chamfered bend with the top view shown in Figure 3.1. By default, there will be an infinite ground plane underneath the substrate. The small rectangles with numbers on them are the ports of the circuit. We can break the circuit into several pieces and connect them to form the circuit as shown in Figure 3.2. As connection of polygons will be discussed in the next chapter, we will construct the chamfered bend as a single polygon in the following. Mark the x and y-coordinates as shown in Figure 3.1. The minimum length is 0.025mm because we can best fit the circuit into a uniform grid of size 0.025 mm. IE3D is a non-uniform grid based simulator. The introduction of the uniform grid here is just for efficient mouse input only. The uniform-grids will not be used in the meshing of the structure in simulation.

Figure 3.1 A chamfered microstrip bend with parameters.

Step 1	Run the MGRID by double-clicking the MGRID icon in the ZELAND FOLDER group. You can also run it from the corresponding icon on the Zeland Program Manager (ZPM or ZELAND.EXE).

Step 2	Select New in File menu.

Explanation:

Before we can enter polygons for a circuit, we need to enter the basic parameters first. The basic parameters include the length unit, layout parameters, the substrate parameters, the metallic strip parameters and the discretization parameters. When you select New in File menu, MGRID will automatically prompt you for the Basic Parameters (see Figure 3.3).

Figure 3.2 The chamfered bend broken down into three polygons.

Figure 3.3  The Basic Parameters dialog.

Figure 3.4 The meaning of the buttons for the listboxes in Figure 3.3.

Explanation:

The Basic Parameters include 6 groups of parameters: (1) Comment: The comment for the whole structure; (2) Length: the Length Unit and Minimum Length for the structure; (3) Layouts and Grids: the parameters for the grid system for the layout editing; (4) Meshing Parameters; (5) Substrate Layers; (6) Metallic Strip Types.

For our structure, we will use “mm” for the length unit. We can accept the default Minimum Length.

No layout and grid are defined yet. We will define the layouts and grids in the next step. Before we do it, we need to understand the buttons above some listboxes in Figure 3.3. The meanings of the buttons are shown in Figure 3.4.

Step 3	Select the “Insert” button for the Layouts and Grids listbox in Figure 3.3.

Response:

Prompt for the Edit Layout and Grid dialog (see Figure 3.5).

Explanation:

The Layout parameters “X-From”, “Y-From”, “X-To” and “Y-To” do not represent the size of the circuit. On the default mode, IE3D’s circuit size extends to infinity in the X and Y-directions. The Layout Parameters only define some range for the editing reference purpose. The default Grid Size = 0.025 mm and it will define a uniform grid system with the grid size = 0.025 mm. A user should understand that this grid system is for geometry construction only. It will not be used for the meshing and the numerical calculation.

Figure 3.5. The Edit Layout and Grid dialog.

Step 4	Select OK to accept the default values for the Layout and Grid.

Response:

The Layout and Grid will be added into the Layouts and Grids listbox. If we like, we can select the “Insert” button above the Layouts and Grids listbox to add more Layouts and Grids. We can switch a set of Layout and Grid during the layout editing.

Step 5	Change the Meshing Frequency (Fmax) from 1 to 40 GHz because we want to simulate the structure up to 40 GHz. Remain the Cells per Wavelength (Ncell) as 20. Un-check the Automatic Edge Cells checkbox. The Automatic Edge Cells are used to enhance the accuracy. We temporarily disable it for faster simulation. Check the Meshing Optimization checkbox. The Meshing Optimization

Explanation:

The higher the discretization frequency is, the shorter the wavelength is, and the finer a circuit should be discretized. Higher discretization rate is preferred for higher accuracy, but you pay for the substantial increase in the simulation time. Many people are concerned about the meshing of a structure, because they understand the simulation results of some electromagnetic simulators are very sensitive to the meshing. IE3D is a method of moment simulator. Simulation results usually are very stable as long as we use about 15-20 cells per wavelength and use small cells along the strongly coupled edges. We will discuss accuracy issues in Chapter 12. We have an option for Automatic Edge Cells (AEC). The AEC improves simulation accuracy significantly. It can yield expert results for novice users. Again, when you enable 
AEC, the simulation time will be much longer. The Meshing Optimization is used to eliminate irregular cells created in the meshing process. Irregular cells are those long cells with tiny area. They always give trouble in numerical analysis. Normally, you should enable Meshing Optimization except for the cases you do not want the meshing program to change the manual meshing you create.

Some users may wonder why we do not implement adaptive meshing. Our experience tells that adaptive meshing is really not necessary, or may even over kill a MOM simulation. IE3D uses a very stable MOM algorithm. For structures with slowly varying frequency response, regular non-uniform meshing is good enough. For structure with fast varying frequency response or structures with strong coupling, regular non-uniform meshing plus AEC will guarantee the accuracy. For narrow bandwidth structures, the simulation resonant frequency may be off 0.1% and it is difficult to eliminate the 0.1% error. However, if you use adaptive meshing, you will end up with a never-ending iteration process, because 0.1% error in resonant frequency may correspond to more than 10-dB difference in the s-parameters at some frequency. Any way, whatever the adaptive meshing can do, the regular non-uniform meshing and AEC can do better. Whatever the AEC on IE3D cannot do, the adaptive meshing cannot do either.

We mentioned that 20 cells per wavelength are normally accurate enough. This rule of thumb is for normal microwave structures. It is not true for low frequency structures or special structures. For digital circuits, the structure is too small compared to wavelength that we need to increase the Fmax such that the structure is meshed into more than a few cells. For MIM capacitors, we even need to mesh the strongly coupled plates into very fine cells in order to get accuracy result. The key is that we need to add more cells at where the current is changing fast. For structures much shorter than wavelength, we also need to mesh it finer in order to get high accuracy result.

# Section 2. Substrate Parameter Definition

Substrate parameters include the number of dielectric substrate layers, the z-coordinate of the dielectric top surface, the substrate permittivity, permeability and conductivity. Complex permittivity, permeability and conductivity are accepted in the simulation. The No.0 substrate layer is assumed to be the ground plane by default. The infinite ground plane is defined as a substrate with a very high conductivity. The z-coordinate of the top surface of No.0 substrate layer is always equal to 0 and can not be modified. Other parameters can be modified according to the actual cases. For example, you can define the conductivity of the No.0 layer as 0 in order to remove the default ground plane.

You can define as many dielectric layers as you like, and you have to define at least one dielectric layer in the upper half space. It means there should be at least 2 dielectric layers (including the No.0 layer, or the ground plane) in a structure. When the upper half space is filled with a single type of dielectric, you can define a dielectric layer with the z-coordinate of top surface as a very large number, such as 1.0e+10 mm. Examples of defining dielectric substrates are documented in Appendix T.

By default, MGRID creates 2 dielectric layers for you. The No.0 layer is a good conductor with conductivity = 4.9e+7 s/m, which is gold. The No.1 layer is air and its top surface Z-coordinate, Ztop = 1.0e+15 mm, meaning that the whole upper space is filled with air.

For the circuit we are going to simulate, we have three dielectric layers (including the ground plane). The No.0 layer is the ground plane. The No.2 layer is the upper half space air, which can be omitted. The No.1 layer is the dielectric slab with the following parameter:

Top Surface Z-Coordinate, Ztop = 0.1 mm

Real Part of Permittivity, Re(EPSr) = 12.9

Loss Tangent of EPSr = 0.0005

Real Part of Permeability, Re(MUr) = 1.0

Loss Tangent of MUr = 0.0

Real Part of Conductivity = 0.0 s/m

Imaginary Part of Conductivity = 0.0 s/m

Dielectric permittivity is also called dielectric constant. There are 2 ways to define substrate loss in IE3D: (1) defining the imaginary part (or loss tangent) of the permittivity and (2) defining the conductivity. Theoretically, they are interchangeable, and they are defined as:

rc = r - j  / ( 0 ) = r ( 1 - j tan)	(3-1)

where r is the real dielectric constant;  is the conductivity;  is the angular frequency; 0 is the complex dielectric constant in free space (8.86  10-12 F/m); tan is the loss tangent. In the other word, we have,

tan = -Im(rc) / Re(rc) = - j  / ( 0 r )	(3-2)

The Loss Tangent is always a non-negative number and the imaginary part of permittivity is always a non-positive number. In practically applications, users always use either tan or  as frequency independent values. However, we can see from (3-2) that, when tan is frequency independent,  will be frequency dependent, or vice-versa. In order to resolve this inconsistency, we define the tan and  in (3-3):

rc = r ( 1 - j tan)- j  / ( 0 )	(3-3)

Users should understand that this is not the true definition of the tan and . It is just for the convenience of users. If a user wants to define the material with frequency independent tan, he should define the  = 0 in the IE3D dialog. If he wants to define the material with frequency independent , he should define tan in the IE3D dialog. However, if he defines non-zero values for both tan and , none of the two values are the true values of the parameters. Defining non-zero for both values is a good way to define frequency dependent tan or . In the near future release, we will even implement user defined frequency dependent tan and  for more flexibility.

You may notice the Conductor Assumption Limit (CAL) is displayed in the. This parameter is defined in the Optional Parameter in Param menu (Appendix A). Inside the IE3D, highly conductive substrate is treated different from the normal substrate. The CAL is used to identify which substrate will be treated as high conductivity substrate in the IE3D. Each substrate will be displayed with a CFactor. If the CFactor of a substrate exceeds the CAL, the substrate will be considered as high conductivity substrate.

Step 6	Select the “Insert” button (see Figure 3.3 and Figure 3.4) above the Substrate Layers listbox. MGRID will prompt you for a new substrate layer (see Figure 3.6).

Step 7	Make sure it is Normal type. Enter the parameter of the No.1 substrate into the dialog (see Figure 3.6) and select OK.

Response:

The No.1 dielectric layer is created and listed in the list box.

Explanation:

A user can select HTS II type for the substrate. We will explain the HTS II type when we discuss the metallic strip parameters in the next.

Figure 3.6 The Edit Substrate dialog with the No.1 substrate parameters entered.

# Section 3. Metallic Strip Parameter Definition

We are going to define the metallic strip types for the structure. For normal conductor, printed strip parameters includes the strip thickness, permittivity, permeability and conductivity, you need to define at least one metallic strip. When you enter a polygon during the geometry editing, the polygon is always assumed to be of the first printed strip type. To modify the printed strip type of a polygon, you select the polygon you want to change the printed strip type, select menu item Object Properties in Edit menu.

You can also define HTS I, HTS II and thin film resistor for the metallic strip types. The HTS I and HTS II do not mean they are different types of HTS material. It just means we use two different formulas for the HTS modeling. To define a printed strip type, you select “Insert” button above the listbox, the layout editor will prompt you to select a type, and define the parameters.

If you want to define all the polygons you entered and will enter as the second printed strip, you can delete the default first printed strip type. The second printed strip type will become the first printed strip type automatically.

For this circuit, we have one printed strip type. The parameters of the No.1 printed strip type are listed below, and it is the same as the default No.1 metallic strip type:

Strip thickness = 0.002 mm

Real part of permittivity = 1.0

Imaginary part of permittivity = 0.0

Real part of permeability = 1.0

Imaginary part of permeability = 0.0

Real part of conductivity = 4.9e+7 s/m

Imaginary part of conductivity = 0.0 s/m

Step 8	Double click at the No.1 printed strip type in the Metallic Strip Types listbox to check it.

Response:

The Edit Metallic Type dialog comes up and let you edit the parameters (see Figure 3.7).

Figure 3.7 The Edit Metallic Type dialog with the No.1 metallic strip type parameters.

Step 9	We do not need to change the parameters because they are what we want. Select OK and MGRID will show all the Basic Parameters on the dialog. We should get the exact parameters as shown in Figure 3.8.

Figure 3.8 The Basic Parameters dialog with all the parameters defined.

Step 10	Select OK. MGRID will be ready for us to create the polygons for the circuit.

The layout and grid of the MGRID is completely shown now. The blue rectangle is the layout. The small red dots are the grids. The red lines are the x and y-coordinate systems with the cross as the origin (the origin x and y may not be 0). The user should understand that the blue rectangle on the display does not represent the boundary of the circuit. For the open boundary formulation, the IE3D assumes the boundary of the substrate and ground plane (if defined) extend to the infinity. For closed boundary formation, the IE3D allows a user to define the boundary of the enclosure. The blue rectangle for the layout just shows the user where his circuit is located. It does not have any other meaning on the size of the circuit and structure.

There are still some other parameters in the Param menu. We do not need to change the Optional Parameters every time we construct a structure. We will explain the Optional Parameters later when you get more familiar with the IE3D.

Section 4. 2D Polygon Input

A polygon is entered as a set of vertices. There are many ways to enter vertices in MGRID. The simplest way is to use the mouse to draw them. Mouse input can be easy and accurate when there exists a good minimum length in the circuit. Good minimum length means that you can fit the circuit into a uniform grid with the minimum length as the grid size. Unlike some other simulators, the layout grid on the IE3D has nothing to do with a simulation. Smaller layout grid does not slow down an IE3D simulation because we do not need to fit a structure into the uniform grid. We will use mouse to construct the chamfered bend in the following.

Step 11	Select 2D Input in Edit menu. Enter "0.1" for the Z-coordinate. Select OK to continue.

Response:

On the layer window, you will see the black strip shift to the “No.2, 0.1000 (mm)” layer. Meaning that the input focus is on the layer.

Explanation:

Selecting 2D Input in Edit menu is a general way to change the input layer z-coordinate. In fact, we can click on a specific layer displayed in the layer window on the lower right color to change the input layer z-coordinate to the layer.

Now, we can enter the polygon vertex by vertex. We can see from Figure 3.1 that the first vertex is at x=0.0 mm and y=0.1mm. It is 4 grids over the grid origin and on the vertical axis.

Step 12	Move the mouse 4 grids over the origin. You will see x=0 and y=0.1 on the status window at the upper right corner. Click the left mouse button.

Explanation:

You need not to locate the mouse cursor exactly on the grid point at x=0.0 mm and y=0.1mm as long as the cursor is closest to the grid point. MGRID will automatically snap the vertex to the grid. When you move the mouse, you can count how many grids the mouse cursor is from a reference point, such as the origin, and keep checking the coordinates displayed in the status window. If a vertex is entered at an unexpected grid point, you can drop the vertex by clicking the right mouse button or selecting Drop Last Vertex in Input menu. To drop multi-vertices, you can continue clicking right mouse button, or you can select Drop All Vertices in Input menu.

Response:

Vertex 1 of Figure 3.1 is entered at x=0.0mm and y=1.0mm and is displayed as a small point on the window. A line linking the vertex to the mouse cursor is also displayed on the window.

Step 13	Move the mouse 4 grids right to the origin. The status window shows x = 0.1 and y = 0.  Click the left mouse button to enter the 2nd vertex.

Response:

Vertex 2 of Figure 3.2 is entered at x=0.1mm and y=0.0mm and an edge between vertex 1 and vertex 2 is created.

Step 14	Move the mouse 26 grids right to the last vertex. The status window shows x = 0.75 and y = 0. Click the left mouse button.

Response:

Vertex 3 of Figure 3.2 is entered at x=0.75mm and y=0.0mm and an edge between vertex 2 and vertex 3 is created.

Step 15	Move the mouse 3 grids up to the last vertex. The status window shows x=0.75 and y=0.075. Click the left mouse button.

Response:

Vertex 4 of Figure 3.2 is entered at x=0.75mm and y=0.075mm and an edge between vertex 3 and vertex 4 is created.

Step 16	Move the mouse 27 grids left to the last vertex. The status window shows x=0.075 and y=0.075. Click the left mouse button.

Response:

Vertex 5 of Figure 3.2 is entered at x=0.075mm and y=0.075mm and an edge between vertex 4 and vertex 5 is created.

Step 17	Move the mouse 27 grids up to the last vertex. The status window shows x=0.075 and y=0.75. Click the left mouse button.

Response:

Vertex 6 of Figure 3.2 is entered at x=0.075mm and y=0.75mm and an edge between vertex 5 and vertex 6 is created.

Step 18	Move the mouse 3 grids left to the last vertex. The status window shows x=0 and y=0.75. Click the left mouse button.

Response:

Vertex 7 of Figure 3.2 is entered at x=0.0mm and y=0.75mm and an edge between vertex 6 and vertex 7 is created.

Explanation:

Vertices 1 to 7 are sequentially connected, but they still have not formed the polygon. To form a polygon, we need to connect vertices 1 and 7.

Step 19	Select Form Polygon in Input menu.

Explanation:

There are several ways to form a polygon. One way is to select item Form Polygon in Input menu. You can also enter a vertex at the exact location of No.1 vertex. MGRID will prompt for confirmation no matter how close the two vertices are. If MGRID does not prompt you for confirmation, it is likely the connection is invalid.  The 3rd way to form a polygon is to double click the left mouse button.

Response:

A polygon is formed and colored for identification. The color of the polygon is the same as the color strip labeled with z=0.1 in the layer window. It means that the polygon is on the layer with a vertical coordinate z=0.1mm. The final result is shown in Figure 3.9.

Figure 3.9 The entered polygon of the chamfered bend.

Section 5. Defining Ports and Displaying Mesh

Construction of the chamfered bend is finished at Step 19. Now we need to define the ports. The simulation engine will not run without any port on a structure. A port is defined on polygon edges. There are multiple ways to define a port. The “Define Port” command in Ports menu allows a user to select an edge by clicking on it. The “Port for Edge Group” command in Ports allow a users to select a group of edges by windowing them. In fact, the “Port for Edge Group” is more popular one even though the “Define Port” is convenience for some cases. We will use “Define Port” in this example. There are other ways to define ports, which we will discuss later in the manual.

Step 20	Select item Define Port in Port menu.

Response:

Prompt for the de-embedding scheme for the port(s) to be defined.

Explanation:

Every port is tied with a de-embedding scheme in IE3D. Different port may need different de-embedding scheme. One de-embedding scheme can only solve some kind of problems. To make the IE3D more flexible, we implemented total 6 de-embedding schemes: (a) Extension for MMIC; (b) Localized for MMIC; (c) Extension for Waves; (d) Vertical Localized; (e) 50 Ohms for Waves; (f) Horizontal Localized. The extension de-embedding schemes, including (a), (c) and (e), are the most accurate ones. They use additional de-embedding arms on the ports to eliminate the high order modes created in the excitation region. However, they require additional space for the extensions. Localized schemes, include (b), (d) and (f), are good for highly packed structures where there is no room for the port extensions. Extension for MMIC is the most flexible extension scheme because it can be used almost in any case. Its accuracy may degrade for high dispersion systems, which we will discuss in Chapter 12. For the 100-micron thick GaAs circuit, the Extension for MMIC scheme can be used below 50 GHz for microstrip structures. For the same substrate, the Extension for MMIC scheme will still be very accurate for differential feed (coupled microstrip lines or CPW) no matter how thick the substrate is because differential feed structures normally have much less dispersion even with thick substrates.

Extension for MMIC can be used for differential feeds, coupled ports and structures without infinite ground plane. The Extension for Waves scheme is the most accurate one for microstrip and stripline structures. However, it is not compatible with the EEsof  Touchstone format. All the other 5 schemes are compatible with the EEsof  Touchstone format. 50 Ohms for Waves is the best for MMIC circuit and filter design. However, it can only be used for isolated ports with infinite ground plane. It cannot be used for differential feeds and coupled ports. That is the reason we assume the Extension for MMIC as the default scheme. The Vertical Localized and Horizontal Localized schemes are extremely flexible localized schemes. They are differential ports by default. The Vertical Localized can completely replace the Localized for MMIC scheme. However, it is not as easy to setup and we still keep the Localized for MMIC for simplicity. More discussion will be available in Chapter 12.

Step 21	Select Extension for MMIC de-embedding scheme and select OK to accept the default “3” cells on the de-embedding arm.

Response:

The MGRID is setup for defining port mode. The default length for the port extension (or de-embedding arm) is 3 cells, and it can be changed to other number in the Optional Parameters dialog in Param menu. For better accuracy, you may increase the number of cells for the extension. We have found that 3 cells normally can yield very good and stable results. However, there are exceptions. If the port extension (displayed as gray color) is not significantly large than the strip to ground distance, you should consider define more cells on the extension.

For higher accuracy simulation, users may want to increase the meshing frequency and number of cells per wavelength. It will increase the cell density. The simulation accuracy normally is higher. However, it is not always true. The port extension, which is fixed with the number of cells, will become shorter due to smaller cells. In order to get higher accuracy, you should increase the cells on the extension

Step 22	Move the mouse to the edge formed by vertex 6 and vertex 7 of Figure 3.9 (or Figure 3.1) and click the left mouse button.

Response:

The port 1 is defined. The edge formed by vertex 6 and vertex 7 is thickened, and a number "1" is printed in a small rectangle on the edge.

Explanation:

The Define Port is layer sensitive. In case, your 2D input layer is not correct, MGRID will not be able to find the edge. In case you see “No edge is selected for the port…”, it is likely that your 2D Input is on a different layer than z = 0.1 mm. You should click on the No.2 (z=0.1 mm) layer on the layer window first and select the edge again.

Step 23	Move the mouse to the edge formed by vertex 3 and vertex 4 of Figure 3.1 and click the left mouse button.

Explanation:

When port 1 is defined, MGRID will still be in the Define Port mode. You can define more ports when necessary.

Response:

Port 2 is defined. The structure will look like what is shown in Figure 3.10 except the meshing is not there.

Step 24	Select Exit Port in menu Port to resume 2D Input mode (You can also select the bitmap for defining port in the toolbar to exit from Define Port mode).

Explanation:

It is a good habit to exit from Define Port mode whenever you finish defining ports. Otherwise, MGRID is always in Define Port mode, and you would not be able to do other process.

Step 25	Select Save in File menu, and enter "c:\ie3d\practice\c_bend.geo" for the geometry file name.

Explanation:

A user can always save part of a circuit no matter whether the circuit is finished or not. It is always recommended to save changes frequently when constructing large circuits. It will avoid accidental loss of data. A user may be curious about how it looks after a circuit is discretized or he may want a hard copy of the circuit layout and the discretized circuit for documentation.

Step 26	Select Display Meshing in Process menu.

Explanation:

The user should be informed that the discretization process is always performed in the simulation engine. The meshing process here is just to give the user an idea how the circuit looks like after it is meshed, and it is not a necessary step. It is highly recommended that a user save the circuit before starting the meshing. The meshing process is a recursive process and it may take much memory and may cause memory problem for large structures if you do not have enough RAM. On the MGRID 9.0, more information on the geometry is provided to the users.

Response:

Prompt for change of highest application frequency and the cell size. Automatic Edge Cells and meshing optimization (see Figure 3.10).

Step 27	Remember to un-check the Automatic Edge Cells option, which we will discuss later. Select OK to accept the default values. A pop-up window comes up to indicate the meshing process.  After the process, MGRID will prompt 21 cells are created with maximum cell size of 0.142 mm in the meshing process. Select Continue and the meshed structure will be displayed in Figure 3.11.

Explanation:

The meshed cell density is determined by the product of the Fmax and Ncell, as well as the AEC. If a user needs to change the cell density, he can change either Fmax or Ncell for the same result as long as the product of Fmax and Ncell is the same. If a user chooses a too big Fmax and/or Ncell, or the circuit is too big such that the estimated minimum cell number exceeds the warning limit, MGRID will issue a warning on it to prevent the case of endless meshing due to the large number of cells created.

After the meshing, the meshed structure will be displayed once. If you want to keep the meshed view until the geometry is changed, you can select the option of “Keep Meshing”.

Figure 3.10  The Automatic Meshing Parameters dialog in Display Meshing.

Figure 3.11 The meshed geometry with port extensions.

Explanation:

Starting from IE3D 9.0, the cells on the de-embedding arms are also shown. There are 4 cells instead of 3 cells as it is specified in Step 21. The additional cell is for the source. As you can see, rectangular cells are used in the regular region and the triangular cells are used in the discontinuities. The efficiency of rectangular cells and the flexibility of triangular cells are smartly used in the IE3D.

If a user wants to get a hard copy of the meshed geometry, he can select Print in File menu to print it out from the printer connected to this computer. For a bitmap picture of the meshed geometry, a user can select ALT+PrtScr buttons to capture the window into the clipboard. Then, he can paste it onto the Paint program in the Accessory group or other graphic application programs.

Section 6. Electromagnetic Simulation

We have constructed the circuit. The next step is to perform an electromagnetic simulation. We will show how to setup a simulation in this section.

Step 28	Select Simulate in Process menu

Response:

The Simulation Setup dialog comes out (see Figure 3.12)

Explanation:

Starting from MGRID 7.0, the simulation setup is performed in one single dialog. The simulation setup data you entered before will be kept for this simulation. Simulation setup and optimization setup share the same dialog too.

There is a Capture button for you to capture the frequency points from earlier simulation results saved in the .sp (.spt, .s2p or .spm) files. There is another button called “Retrieve”. It allows you to retrieve most of the data for the Simulation Setup dialog from some previous simulation input file (with extension .SIM). The data retrieved including the frequency points and optimization objectives (for optimization), etc.

The meshing parameters are important ones and they are also accessible from this dialog. You also need to enter the frequency points for the simulation.

Moment method simulator such as the IE3D needs to solve matrices. The time for solving matrices is the most important part in an IE3D simulation. We have implemented multiple matrix solvers for different purposes. The default matrix solver is the Adv. Symmetric Matrix Solver (SMSi). The symmetric matrix solvers SMSi and SMS are the general purpose and efficient matrix solvers, and the SMSi is much faster than the SMS. However, the SMSi matrix solver is only available on the Intel Pentium Pro, Pentium II, Pentium III, Pentium 4 and AMD Athlon CPUs. The SMSi matrix solver will crash on earlier CPUs. If you try this example and it crashes on you, it will be 100% sure that your computer has an earlier CPU. You should choose the SMS matrix solver. You can define the default matrix solver in the Optional Parameters in Param menu. More discussion on matrix solvers is available in Chapter 12

In most cases, the users are interested in the frequency responses of the s- (y-, z-) parameters. If you are simulating the structure for the complete frequency response, you should always enable the Adaptive Intelli-Fit (AIF). The AIF is an extremely efficient and robust way to get fast and accurate frequency response. However, if you enable the AIF, you will not be able to enabling saving the current distribution file and calculation of the radiation pattern, which we will discuss later. For antenna applications, we certainly are interested in the current distributions and radiation patterns. For such a case, we should simulate the structure at many frequency points using the AIF. Then, we can re-run the simulation at the frequency points of interest, such as those resonant points with AIF disabled. We can enable saving the current distribution and/or calculation of the radiation pattern in the 2nd simulation at the specific frequencies.

For small and medium size structures, the IE3D simulation is almost interactive. You should check the Auto Invoke option in the Post-Processing group of the dialog. The IE3D will invoke the MODUA simulator for display and other post-processing after the simulation is done.

After the simulation setup, you have 4 options: (1) Invoke IE3D; (2) Create .SIM file only; (3) Append to Queue; (4) Invoke IE3D for Net. For interactive simulation or for one single long simulation, you may want to use the Invoke IE3D option. It will invoke the simulator to perform the simulation immediately.  The 4th option is for network distributed IE3D computing. The users need to have the ZDS/ZDM licenses in order to use such a feature.  We will discuss the ZDS/ZDM and network distributed IE3D computing later.

There are two accuracy options: 2D simulation option with For Speed and For Accuracy, and 3D simulation option with Normal and For Accuracy. They are automatically set by MGRID. Users should not change them normally. The For Speed is normally for structures in free space and the For Accuracy is normally for structures in layered dielectrics.

Other parameters in the dialog are for tuning and optimization. Both cases require the user to define some optimization variables. We will discuss them when we use tuning and optimization.

Step 29	Enter “0.5” for the Start Freq (GHz), “40” for the End Freq (GHz) and “80” for the Number of Freq. Hit the Enter key on the keyboard and select ENTER in the dialog.

Response:

Total 80 frequency points from 0.5 to 40 GHz are entered (see Figure 3.12).

Figure 3.12  The Simulation Setup dialog on MGRID.

Explanation:

Some users familiar with other electromagnetic simulators may not feel comfortable to define so many frequency points in the beginning. Electromagnetic simulation is normally a time consuming process. Some electromagnetic simulators may take a long time for each frequency. For the IE3D, you do not need to worry about it. IE3D is a very fast and accurate simulator. The AIF will make it even must faster. With the AIF enabled, you can define as many frequencies as you like. It will not slow down the simulation. In fact, you are suggested to define enough points in the frequency band for smooth fitting.

Step 30	Make sure all the other parameters are as shown in Figure 3.12. Select OK to continue.

Response:

The IE3D simulation engine (the IE3D.EXE) is invoked to perform the simulation (see Figure 3.13).

Figure 3.13. The IE3D simulation engine’s dialog.

Explanation:

The Simulate process is finished. All the simulation data is saved into the simulation input file: c:\ie3d\practice\c_bend.sim. This data file will be used as the command-line argument in calling the simulator IE3D.EXE.

When you select OK, the IE3D simulation engine IE3D.EXE will be invoked automatically to perform the simulation. The internal command is in the form:

ie3d  “c:\ie3d\practice\c_bend.sim”	(3-4)

The double quotation marks should be used to prevent ambiguity of space in the path name. Starting from IE3D 9.0, The IE3D engine is a dialog-based application. It will provide more information to the users, while it preserves all the features of the console IE3D engine in the earlier versions.

We should discuss the After Setup option for the Simulation Setup dialog now. We have 3 options: (1) Invoke IE3D; (2) Create .SIM File Only; (3) Append to Queue; (4) Invoke IE3D for Net. If we choose Invoke IE3D, the IE3D simulation engine will be invoked immediately to perform simulation. If you select Create .SIM File Only, the .SIM file in (3-4) will be created. However, the command will not be executed. A user can perform a few simulation setups for a few structures. Then, he can write a batch file (with extension .bat) to run the a few simulations sequentially. Each line of the batch file should be similar to (3-4). If you have not built a path to the IE3D simulation engine, you should also include a path for it. The default directory for the IE3D.EXE is c:\program files\zeland\exe. You should write the command line as:

“c:\program files\zeland\exe\ie3d”   “c:\my_directory\my_sim_file.sim”	(3-5)

The double quotation marks are for file names with space. It is always safe to use double quotations for the file names. Otherwise, any space in the long file name will confuse the OS.

The third option is Append to Queue. Basically, the newer versions of the IE3D allow users to create a simulation queue on the Zeland Program Manager. The Zeland Simulation Queue will take care of the scheduling of your simulations for you automatically. This feature allows a user to submit jobs to different IE3D licenses on different machines in the network. Interested users please read the notes on the Zeland Program Manager. To submit a simulation job to the Zeland Simulation Queue is simple. You just define all the parameters, select Append to Queue, and select OK. You may not see anything happens immediately. However, the job will be submitted to the queue. It will be run by any IE3D license using the same queue.

The fourth option is Invoke IE3D for Net. It is for the distributed IE3D computing and optimization. We will discuss distributed computing in the Appendix Z.

Figure 3.14 The Smith Chart display of the frequency response.

Step 31	The IE3D will finish the simulation in less than one second on modern computers. It will show all the information on the IE3D simulation engine dialog (see Figure 3.13). After it is finished, the MODUA simulator is invoked automatically to display the data in Smith Chart or in other forms depending upon the setting in the Optional View Settings in the View menu of MODUA. Figure 3.14 shows the Smith Chart display on MODUA. Please minimize or close the MGRID window.

Explanation:

When the IE3D finishes the simulation, it will save the result into the file: c:\ie3d\practice\c_bend.sp. Then, it will invoke the MODUA to display the parameters. If you do not want the MODUA to be invoked, you should un-check the Auto Invoke in the Post Process in the Simulation Setup dialog discussed in Step 29.

For new users, they may take the automatic display granted. We would like to illustrate how we can display the data manually. It will be helpful for you to understand the important features of the IE3D.

Here are some more comment on the .sp and the .log files. The .sp file contains the results in the HP/EEsof  Touchstone  compatible format. However, we do not want to use the same .s2p extension. It is inconvenient to put the port information into the file extension. The .log file contains the simulation intermediate data. If a user wants to check some intermediate data for the above simulation, he should check the file: c:\ie3d\practice\c_bend.log.

In case an error is encountered, the IE3D engine normally should inform you. You can also check the .log file. If the .log file is not created, you may want to open the Command Prompt (start\programs\command-prompt) and run the simulator under the Command Prompt as in (3-2). The intermediate information will be printed out from the Command-Prompt window. Starting from IE3D 8.0, the created .log file will be opened on Notepad if there is any error encountered.

It is possible that the user may not get the Smith Chart display window if the default setting on MODUA is not for Smith Chart display. To change the default display settings on MODUA, a user can chooses the Optional View Settings in the View menu of MODUA.

Section 7. Graphic Display of Parameters and Curve-Fitting

Graphic display is performed on the schematic editor MODUA. In the last example, MODUA is invoked to perform the display for you. In fact, you can do it manually. The automatic way saves you some steps. However, in order for you to understand the detail, we will demonstrate how we can manually use the MODUA, the integrated circuit simulator, to process data.

Step 1	At this time, the MODUA is still displaying the data. You can close it and re-run the MODUA from the ZELAND FOLDER. However, you can just select the Display Parameter Module in File menu.

Response:

Prompt for the parameter file name.

Step 2	Select file c:\ie3d\practice\c_bend.sp and click on OK.

Response:

MODUA reads the data and display the S11, S21 and S22 on a Smith Chart.

Explanation:

The picture will be similar to what is shown in Figure 3.14. This should be what you saw in the last step of the last section. If you choose Auto Invoke in the Post Process in the Simulation Setup dialog of MGRID, the IE3D will do the steps 1 and 2 of this section for you automatically. In fact, the Display Parameters Module command is already a simplified command for MODUA. The MODUA is a standard network simulator. It allows a user to use the Add Parameters Module, Add Geometry Module, and whole Element menu to build a design consisting of different elements. Then, you can perform a frequency domain simulation including s-parameters, IE3D geometry files and lumped elements. The Display Parameter Module command avoids the tedious design setup procedures, when you are using the MODUA for display of s-parameters.

While you are displaying the s-parameters in Smith Chart, if you want to frequency response in other format, you can select the Define Display Data or Define Display Graph in Control menu. If you want to see the design setup, you can select the Display Toggle in Control menu. The design consists of 3 modules: the c_bend module, the port 1 and port 2 modules.

By default, the Smith Chart is displayed when users choose the Display Parameter Module. However, if a user likes a different default, he can setup it up on the Optional View Settings in the View menu.

Step 3	Select Display Parameter Module in File menu.

Explanation:

If we had not enabled the Adaptive Intelli-Fit (AIF) in the simulation setup, the result in c:\ie3d\practice\c_bend.sp would be the result of the original electromagnetic simulation. Since we have checked the Enabled button for the AIF. The result in c:\ie3d\practice\c_bend.sp is derived from c:\ie3d\practice\c_bend.spt by a special interpolation scheme.

Some users may be concerned about the fact that the results in c_bend.sp are the results of interpolation when the AIF is enabled. They may worry about its accuracy. Our recommendation to those users is that you do not need to worry about it. A large number of validation simulations were performed on the AIF in our lab. Almost 100% of the AIF results are perfect agreement to the original results. It can capture all the detail of a multi-resonant antenna or circuit in a wide frequency range using few frequency points. Also, a large number of users are using the IE3D every day and no failure report was received.

However, we do want to warn those users with high performance and narrow-band (such as 0.5%) filter designs. For those applications, the pass-band properties may look just like a square waveform. It is almost flat in the pass-band and it transit to stop band in an extremely narrow frequency range. The users should try to concentrate the Adaptive Intelli-Fit to a range covering the pass-band and its vicinity only. For such a filter, if you try to use the Adaptive Intelli-Fit to cover a wide frequency range. The pass-band may create. Eventually, the AIF will fail due to the finite word length of numerical simulation.

Step 4	While MODUA prompts you for the parameter file, please change the Files of Type from Parameter Files (*sp) to Parameter Files (*.spt). The files with extension .spt will be listed. Please select c:\ie3d\practice\c_bend.spt.

Response:

MODUA will display the frequency response of the c:\ie3d\practice\c_bend.spt.

Explanation:

It is the original electromagnetic simulation result. We will see a picture similar to the Figure 3.14. However, there are just few frequency points.

Step 5	Select Graph Parameters in View menu.  Make sure you check the Display Frequency checkbox and the Interval is 1 (see Figure 3.15). Select OK to continue.

Figure 3.15 The Graph Parameters dialog from the View menu of MODUA.

Response:

The curves in the Smith Chart are re-displayed with the frequencies labeled as: 0.5, 3.5, 8, 10.5, 20.5, 25.5 30.5 and 40 GHz.

Explanation:

The electromagnetic simulation is performed only at the 8-frequency points. All the other frequency points are interpolated data.

Step 6	Select Define Display Data in Control menu. Select OK to continue.

Response:

Prompt you to select the parameter type for display.

Step 7	Select “dB and Phase of S-Parameters” in the list box and select OK.

Response:

Prompt you for the display items.

Step 8	Select 4 items: “dB[S(1,1)]”, “ANG[S(1,1)]”, “dB[S(2,1)]” and “ANG[S(2,1)]”  and select OK to continue.

Response:

The frequency response of the selected items will be displayed on the MODUA window. The data at selected frequencies are shown in Table 3.1.

Table 3.1 The frequency response in the c_bend.spt.

| Freq[GHz] | dB[S(1,1)] | ANG[S(1,1)] | dB[S(2,1)] | ANG[S(2,1)] |
| --- | --- | --- | --- | --- |
| 0.5 | -50.99 | 38.88 | -1.86E-2 | -2.374 |
| 3.5 | -38.17 | 62.66 | -2.22E-2 | -16.59 |
| 8.0 | -32.31 | 46.33 | -2.96E-2 | -37.95 |
| 10.5 | -31.04 | 35.41 | -3.52E-2 | -49.86 |
| 20.5 | -34.54 | -5.95 | -7.13E-2 | -97.99 |
| 25.5 | -45.32 | 8.21 | -9.88E-2 | -122.4 |
| 30.5 | -39.26 | 90.11 | -0.125 | -146.9 |
| 40.0 | -40.53 | 52.20 | -0.126 | 166.2 |

Step 9	Select Print in File menu, choose the right printer driver and select OK.

Response:

A hard copy of the data list is printed out.

Step 10	Select Save Displayed Data in File menu.

Response:

Prompt you for the file name with default “c_bend.lst”.

Step 11	Select OK to accept the default.

Response:

The data list will be saved into file c:\ie3d\practice\c_bend.lst”. You can use an editor to open it.

Step 12	Select Define Display Graph in Control menu.

Response:

Prompt you to select the parameter type for display.

Step 13	Select “dB and Phase of S-Parameters” in the list box and select OK.

Response:

Prompt you for the display items.

Step 14	Select 4 items: “dB[S(1,1)]”, “ANG[S(1,1)]”, “dB[S(2,1)]” and “ANG[S(2,1)]”  and select OK to continue.

Response:

The frequency response of the selected items will be displayed in a graph. You may notice the curves are not smooth because we do not have enough frequency points.

Step 15	Select Curve-Fitting and Interpolation in Process menu.

Response:

Prompt for the curve-fitting scheme and the frequency interval for the interpolation.

Explanation:

There are 3 curve-fitting schemes available: Cubic Spline, Intelli-Fit and General Intelli-fit. The Intelli-Fit is an old implementation. It is not very robust. Cubic-Spline is a good scheme. However, it only works well with enough points. The General Intelli-Fit is the same scheme as the Adaptive Intelli-Fit scheme implemented into the IE3D. We should normally always use the General Intelli-Fit scheme.

Step 16	Select GENERAL INTELLI-FIT and select OK to continue.

Response:

Prompt you for the interpolation frequencies.

Step 17	Enter “0.5” for Start Frequency, “40” for End Frequency, and “80” for Number of Frequencies. Select ENTER button. Select OK to continue.

Response:

The curves in the graph become smooth now. The picture will look exactly like what is shown in Figure 3.16. As you can see, only the points we see the markers are actually simulated. Other points are from the interpolation. There are two grids for the y-directions. The dB[S(i,j)] uses the left grid and ANG[S(i,j)] uses the right grid.

Step 18	Select Display Grid Toggle in View menu.

Response:

The display will toggle between whether there is a grid in the graph.

Step 19	Select Design Identification in View menu.

Response:

Prompt for the design identification.

Step 20	Enter “curve-fitted” and select OK.

Response:

The Design Identification “curve-fitted” is displayed in the legends of the display items.

Step 21	Select Save S-Parameters in File menu.

Response:

Prompt for change of the default filename c:\ie3d\practice\c_bend.spm, valid digits and file format.

Explanation:

IE3D uses at least 10 valid digits for s-parameter output. The reason is from the consideration of preserving the accuracy of the simulation result. Some other simulators may not accept so many digits. In such a case, you can use the Display Parameter Module and the Save S-Parameters in File menu to convert to the right one you like. The IE3D simulator uses the Touchstone  format as the default output format. There is some difference between the Touchstone  format and the MIMICAD format for large number of ports. For an N-port s-parameter file in the MIMICAD format, the S(1,1) to S(N,1) is always on a single line, no matter how bigger N is.

Figure 3.16 Linear graph display with gridlines.

Step 22	Select OK to accept the default name.

Response:

The curve-fitted data is saved into file c:\ie3d\practice\c_bend.spm.

Explanation:

The saved result is a curve-fitted result. It is the same as the c_bend.sp data.

Section 8. Displaying Multiple Simulations on One Graph on MODUA.

Step 23	Select Parameter File Queue in File menu.

Explanation:

We are going to compare the curve-fitted result with the original result.

Step 24	Click the Add button. Select c_bend.spt and OK to continue.

Response:

Prompt for the Queue Module ID.

Step 25	Type “original” and select OK.

Response:

The file c_bend.spt is added to the queue.

Step 26	Select Display button. This button is new on MODUA 8.0. It automatically allows you to go to the Display Queue Items in the View menu.

Explanation:

In the old version, we need to exit the Parameter File Queue and select Display Queue Items in the View menu in order to define the items in the queue for display. The Display button on the Parameter File Queue dialog simplifies this process. On the Display Queue Items dialog, you can also see the Queue Files button. It allows you to go to the Parameter File Queue dialog directly.

Step 27	Select the c_bend.spt module in the queue list.

Response:

Prompt you for the display items.

Step 28	Select 4 items: “dB[S(1,1)]”, “ANG[S(1,1)]”, “dB[S(2,1)]” and “ANG[S(2,1)]”  and select OK to continue.

Response:

Total 4 display items are added to the queue module c_bend.sp.

Step 29	Check the “Display Queue” and you will get the picture in Figure 3.17. Select OK to continue.

Figure 3.17 The Display Queue Items dialog in the View menu of MODUA.

Response:

The queue items are displayed simultaneously with the curve-fitted result as shown in Figure 3.18.

Explanation:

As you can see from Figure 3.18, the curve-fitted result matches the original result exactly at the frequency points listed in Table 3.1.quite well. If you still doubt the accuracy of the AIF (or General Intelli-Fit in MODUA), you can simulate the same geometry file without enabling the AIF. You will get the same results.

Figure 3.18 Comparison between the original result and the curve-fitted result

Step 30	Close the MODUA window.

Section 9. High Accuracy Simulation Using the Automatic Edge Cells Feature.

We can claim that the AIF scheme yields “accurate” result over a wide bandwidth by just simulating a few frequency points. However, when we say “accurate”, we do not mean that we have proved the result is accurate compared to the measured data or converged data. Here, “accurate” just means that the result of AIF will match the true simulation results under the same condition perfectly. Whether the simulation results are accurate or not, we need to do many validation by comparison to measured data or converged data. In the past 10 years, we have been validating the IE3D by comparing its results to measured data or by performing convergence studies. We have come up with different rules for accurate electromagnetic modeling using the IE3D. We will discuss different accuracy enhancement methods in Chapter 12 since it may need a user to read more chapters before he can understand the schemes. In this section, we would like to touch base of Automatic Edge Cells (AEC), a simple and efficient way to improve simulation accuracy. It really can let novice users yield expert results.

Step 1	Run MGRID. Open the file “c:\ie3d\practice\c_bend.geo”. Select Save As in the File menu and save the geometry as “c:\ie3d\practice\c_bend3.geo”.

Step 2	Select Display Meshing in menu Process.

Explanation:

We are going to show how the Automatic Edge Cells affects meshing. The Display Meshing process is not necessary for a simulation. We just want to show you the meshing. Otherwise, we can go directly to Step 4.

Response:

Prompt for change of Fmax and the Ncells, AEC and Meshing Optimization.

Step 3	Check the Automatic Edge Cells checkbox. Enter the Width = 0.01 mm. Select OK to continue. The structure will be re-meshed. This time, 69-cells (vs. 21 cells without AEC) are created. Select Continue and the meshed result using AEC is shown in Figure 3.19.

Explanation:

The only important and confusing parameter for AEC is the Width. Some long time IE3D users may also have difficulty to decide how to set the AEC edge Width. The rule of thumb is to set it to be about 10% of the cell size. We know the strip width for the bend is 0.075 mm. We also know the bend will be meshed into 1-cell in the transverse direction. Therefore, the regular cell size is about 0.075 mm. In fact, it is slightly larger. We choose the Edge Width = 0.01 mm which is in the range of 10-15% of the cell size.

Figure 3.19 The meshing of the bend with Automatic Edge Cells.

Step 4	Select Simulate in Process menu.

Response:

The simulation setup dialog comes up. All the frequency points we defined in the 1st simulation are still kept there. This is the new feature in IE3D 7.0. The simulation setup data is preserved. The AEC is still enabled because it is enabled in the last meshing.

Step 5	Select OK to continue.

Response:

The IE3D simulation is performed on the c_bend3.geo. The simulation will be done in 2-3 seconds. After the simulation, the MODUA will be invoked to display the result.

Step 6	Select Define Display Graph in Control menu. Select “dB and Phase of S-Parameters”. Select OK to continue. Select “dB[S(1,1)]”, “Ang[S(1,1)]”, “dB[S(2,1)]” and “Ang[S(2,1)]”. Select OK to continue.

Response:

The graph will be displayed.

Step 7	Select Design Identification in View menu. Enter “3-Cells” and select OK. The design ID will be displayed in the legends.

Step 8	Select Parameter File Queue in File menu. Click the Add button. Select c_bend.sp and OK to continue.

Response:

Prompt for the Queue Module ID.

Step 9	Type “1-Cell” and select OK to add the file into the queue. Select OK to continue.

Response:

The MODUA window is not changed.

Step 10	Select Display Queue Items in View menu. Select the c_bend.sp in the queue list. Select 4 items: “dB [S(1,1)]”, “ANG[S(1,1)]”, “dB[S(2,1)]” and “ANG[S(2,1)]”. Select OK to continue.

Response:

Total 4 display items are added to the queue module c_bend.sp.

Step 11	Check the Checkbox “Display Queue” and select OK to display the results (see Figure 3.20).

Figure 3.20 The comparison between the c_bend.sp and c_bend3.sp.

Explanation:

The S21 agrees very well for both simulations. There is some difference in the S11. The dB[S11] is in the range of –30 dB. We expect the results of the c_bend3.geo should be very accurate. Some users may wonder whether we can get more accurate dB[S11]. They may want to know the results using 5 cells in the transverse direction. We can use the Add Edge Vertices in Edit menu to make the geometry to be meshed into 5-cells in the transverse direction. We will discuss the scheme later in Chapter 12. The geometry c_bend5.geo in the c:\ie3d\samples directory is pre-processed for it. Interested users can try to use AEC with Edge Width of 0.005 mm on the c_bend5.geo structure. Basically, the c_bend5.geo will create 2 small edge cells. Comparison shows little difference between the c_bend3.sp and c_bend5.sp. For most structures, one edge cell is really good enough. You should understand that put more edge cells will slow down the simulation significantly, especially for large structures. After we use 1-edge cell along each edge, the accuracy gain will be small if we put more edge cells.

Section 10.  Simple Ways to Build a Chamfered Bend and Other Path Structures

We discussed how we build the bend in Sections 4 and 5. Basically, the MGRID allows you to enter the bend as a 7-vertex polygon. We do not mind entering the vertices one by one for this 7-vertex polygon. However, if the circuit consists of many bends, it will be tedious to construct it as a polygon with many vertices. There are simpler ways to make it. Let’s discuss them in this section.

Step 1	Run MGRID. Open the file “c:\ie3d\practice\c_bend.geo”. Press down “Shift” button and click the mouse button at the 7-vertex polygon.

Response:

The polygon is selected and turns to black color. The status window will display “Select Polygon Mode, use mouse to select polygons in all layers…”.

Explanation:

In the old versions of the IE3D, the default mode is the drawing mode. To select polygon(s), we need to select the Select Polygon (or Select Polygon Group) in Edit menu first. Then, we click at the polygon (or window the polygon for the Select Polygon Group mode) to select the polygon. Starting from the IE3D, we make the selection procedure easier. We can get onto selection mode by pressing down the Shift button.

We are going to re-construct the bend. We do not need to start from the New command in the File menu. If we started from New, we had to re-enter all the Basic Parameters. We can delete the existing bend in the c_bend.geo file and continue from it.

Step 2	Select Delete in Edit menu to delete the polygon. It is equivalent to selecting the scissors in the tool bar.

Response:

The polygon is deleted.

Explanation:

We are ready to re-enter the bend structure using other ways. We know the bend can be defined as a path with 3 vertices at (x,y) = (0.0375, 0.75), (0.0375, 0.0375) and (0.75, 0.0375). It would be easier if we can just enter the 3 vertices and MGRID can build the path for us. The MGRID does have such a feature.

One thing you may realize is that the 3 vertices are not at the layout grid points. The origin of the layout is at (x, y) = (0, 0). Each grid is of size 0.025 mm. In order to use the mouse input, we have to define a grid system with all the vertices located exactly at the grid points. MGRID allows a user to define more grid systems in the Layouts and Grids section of the Basic Parameters in the Param menu. In this section, we will show the users how we can use keyboard input as an alternative and fast way to enter the vertices off the grids.

Step 3	Click at the color strip “2, 0.10000 (mm)” on the layer window to shift the 2D Input focus to the z = 0.1 mm layer. Select Key in Absolute Location in Input menu. MGRID will prompt you to enter the X and Y-coordinates of the vertex. Enter X-Coordinate = 0.0375. Type the Tab key to shift the focus to the Y-Coordinate edit control. Enter the Y-Coordinate = 0.75. Then, hit the Enter key (or select OK).

Response:

The 1st vertex is entered at (x, y) = (0.0375, 0.75).

Step 5	Type Shift+A. Shift+A means that you press down the Shift key and hold it. Then, you type the A key. Finally, you release the hold Shift key. It is equivalent to selecting Key In Absolute Location in the Input menu.  You will be prompted for the location of the next vertex. The last coordinates of the last vertex are used as the default coordinates. Make sure the X-Coordinate = 0.0375. Then, type the Tab key to shift to the Y-Coordinate edit field. Enter Y-Coordinate = 0.0375. Hit the Enter key.

Response:

The 2nd vertex is entered at (x, y) = (0.0375, 0.0375).

Explanation:

Shift+A is the accelerator for the menu item Key In Absolute Location in the Input menu. It is listed in the right hand side of the menu item. You can access the menu item using the accelerator.

Step 6	Type Shift+R.

Response:

You will be prompted for the offset of the next vertex to the last vertex.

Explanation:

The next vertex should be located at (x, y) = (0.75, 0.0375). The last vertex is at (x, y) = (0.0375, 0.0375). The offset should be (dx, dy) = (0.7125, 0).

Step 7	Enter X-offset = 0.7125 and the Y-offset = 0.0. Hit Enter key.

Response:

The 3rd vertex is entered at (x, y) = (0.75, 0.0375).

Explanation:

Whatever we are doing is just like we are entering the vertices for a polygon. If we select Form Polygon in Input menu, we will create a triangle with the 3 vertices. However, we are not building the triangle. We are building a path along the line connecting the 3 vertices.

Step 8	Select Build Path in Adv. Edit menu. Enter the Path Width = 0.075 and select the Path Style as Smooth Corner (the default). Hit Enter (or select OK) to continue.

Response:

A chamfered bend is created on MGRID (see Figure 3.21).

Explanation:

The bend is in the same location as the one created in Section 4. However, the cut at the corner is smaller. This bend is automatically created. Users do not have the control over some of the small detail. The small difference in the corner normally has little effect to the performance of the structure. You may leave it as it is. However, if you want to make it exactly like the bend we constructed in Section 4, you need to move the vertex 1 in Figure 3.21 in the y-direction in by 0.025 mm, and the vertex 2 in x-direction by 0.025 mm.

Figure 3.21 The chamfered bend created using the Build Path in Adv Edit menu.

Step 9	While you are in the default drawing mode, press down “Shift” and window the vertex 1 only in Figure 3.21. What “window” means here is defined in Chapter 2. Basically, you move the mouse to the upper left corner of vertex 1. Press down the left mouse button. Drag it to the lower right corner of vertex 2. Then, you release the left mouse button.

Response:

A small rectangular marker is displayed at the vertex 1 location, indicating the vertex 1 is selected.

Explanation:

In version older than 9.0, we have to select Select Vertices command in Edit menu first to get into Select Vertices mode before we can window the vertices we want to select. Starting from IE3D 9.0, when it is in the default drawing mode, a user can use “Shift” to directly shift to the selection mode.  For an old user who are used to the old command  styles, he can always follow the old method to select the vertex 1 in this step: select Select Vertices in Edit menu. Then, window the vertex 1 only in Figure 3.21.

Step 10	Press down “Shift”. Press down the left mouse button and move. MGRID automatically gets into the Move Objects (in Edit menu) mode. This is also a new feature on the MGRID 9.0. In the earlier version, a user has to select Move Objects in Edit menu in order to move the selected objects. The Move Objects command still works. However, the new move command we are using here is more convenient. Move the mouse upward somewhere and release the button and the “Shift” key. In fact, we can release the “Shift” key after we press down the left mouse button.

Response:

The selected vertex is moved upward somewhere. The “Move Object Offset to Original” dialog comes up and display the offset values (Figure 3.22).

Explanation:

MGRID is a very precise layout editor. The coordinates of each vertex of the polygons are described precisely using double precision floating point values. When we use the move and copy commands, we need to preserve the precision. Therefore, we always prompt the users to confirm the coordinates on the move and copy commands. The users can always enter the expected values they want.

There are in fact 3 snapping modes for the move and copy commands: (1) Snap to Vertex, (2) Snap to Edge, (3) Snap to Vertex. The default snapping mode is “Snap to Vertex”. To change the snapping mode, a user can type F4 while he is moving the vertex. If the user uses the Move Objects in Edit menu command to move the objects, he does not need to keep the left mouse button pressed, in this case, he can select “Change Snapping” in Edit menu (corresponds to F4) to change the snapping. In the Move Objects mode, a user can also change the Moving Reference Vertex by clicking the right mouse button or use the “Set Moving Reference Vertex” in Input menu. The Moving Reference Vertex is the vertex following the cursor in the move and copy command.

For our current moving, there are not vertices near the vertex 1. It will not snap to any vertex. We do not need to worry about because we know the offset values as: dx = 0 and dy = 0.025 mm.

Figure 3.22  The Move Object Offset to Original dialog.

Step 11	Enter the X-offset = 0 and the Y-offset = 0.025. Hit Enter to continue.

Explanation:

While we are entering the offset values, you will see the After Move option changes form “Objects Remain Selected” to “Objects De-Selected”. This option allows the moved objects to be still selected or de-selected for further editing. For our case, we do not need to apply more commands to the vertex 1 and we just accept the changed default setting of “Objects De-Selected” After Move.

Response:

The vertex 1 is moved in the y-direction with Y-offset = 0.025. The result is shown in Figure 3.23.

Step 12	Press down “Shift” and window the vertex 2 only in Figure 3.21 to select it. Press down “Shift” and press down the left mouse button and move the mouse to move the vertex 2. The “Move Object Offset to Original” dialog will come up. Change the X-offset = 0.025 and the Y-offset = 0, and hit Enter (or select OK) to continue.

Response:

The vertex 2 is moved with the offset as (dx, dy) = (0.025, 0).  MGRID resumes to the geometry entry mode automatically because the changed default for After Move is Objects De-Selected.

Explanation:

The shape of the bend after Step 14 will be exactly as the bend we want to build (Figure 3.24). Certainly, we built the bend as a single polygon in Section 4. Here, we built it as 3 polygons using the Build Path command and moved vertices. The results of both structures will be identical if the meshing on them is identical. In case the different builds create different meshes, we cannot expect them to be identical due to the numerical feature of the simulator. However, the numerical error is expected to be extremely small.

Figure 3.23  The shape of the bend after the vertex 1 is moved.

Figure 3.24. The shape of the bend after the vertex 2 is moved to the correct location.

Step 13	Save the geometry file as c_bend1.geo.

There is still another simpler way to build the chamfered bend. It is using the Chamfered Bend in the Entity menu that will be documented in the Appendix I. Interested users can try it out.

When we build the structure as 3 polygons. We need the 3 polygons to be connected “electrically”. We have built the 3 polygons using automatic way. The “electrical connection” among the 3 polygons is guaranteed. The guaranteed electrical connections are indicated by the red dots at the centers of the common edges (see Figures 3.23 and 3.24). In case we need to build the connected polygons, how can we guarantee the “electrical connection” between them? Polygon connection or “electrical connection” is a very important concept in the IE3D. We will illustrate it in the next chapter.