Chapter 2   Basic Concepts and Window Configuration

Before going through any example, we will first explain some important concepts, and explain the basic configuration of the MGRID, MODUA and CURVIEW windows.  The PATTERNVIEW window is self-explanatory and its parameters have much similarity to the CURVIEW. We will not give detailed explanation for PATTERNVIEW.

Section 1. Basic Concepts

In this section, we will explain some basic concepts frequently encountered in the following chapters.

AEC ----------------------	Automatic Edge Cells, a feature to add small cells along the edges for guaranteed simulation accuracy.

AIF ------------------------	Adaptive Intelli-Fit, a scheme allows user to get the frequency response of many frequency points with guaranteed accuracy by simulating just a few frequency points.

Cell ----------------	---------	A small rectangle or triangle in a mesh.

Current Data File-------	The file created by IE3D for use in CURVIEW. It takes .CUR extension.

Cursor -------------------	The pointer of the mouse.

De-embedding Arm ---	The extension on each extension port. It is created automatically by the simulator to stabilize the standing waves on the structure.

Design File ---------------	The file created and saved on MODUA. It takes .DSG extension.

Edge  --------------------	An edge is a straight line formed by connecting two consecutive vertices of a polygon.

Electrical Connection

----------------	-------	Two polygons are electrically connected if they have a common edge.

Fmax  ----------------------	The Meshing Frequency used to mesh the structure. It is also called the highest application frequency.

Geometry File------------	The file created and saved on MGRID. It takes .GEO extension.

Grid  -----------------------	A uniform mesh on the MGRID window for ease of mouse input.

Gridding -------------------	See Meshing.

Meshing -------------------	The process to partition polygons into rectangular and triangular cells.

Minimum Length

---------------------------	The largest length value of a grid with all the dimensions of a circuit fitting into this grid. Example: The minimum length of a rectangular strip of 2 mm by 5 mm is 1 mm.

Mouse ---------------	----	An input device. Two mouse buttons, left button and right button, are utilized in MGRID for selection and data input. The left button is used to select objects and menus. When it is in polygon input mode, the left button is generally used to locate a vertex. The right button is used to delete the last vertex. Double clicking the left button causes a set of vertices to form a polygon.

Ncell ---------------------	Cells per Waveguide Wavelength. It is the meshing parameter. It controls how fine the structure is meshed into cells.

Node --------------	---------	A vertex of a cell.

Optimization ------------	The process of tuning the shape of a circuit by the simulator to achieve an optimized circuit of required performance.

Pattern File -------------	The file storing pattern information created by IE3D or MGRID. It should be used by PATTERNVIEW.

Polygon -----------------	A set of vertices connected one by one to form an enclosed region. The interior of a polygon is colored on the MGRID window.

Port ----------------	--------	A port is a terminal(s) of a circuit. On the IE3D, a port is one terminal of the structure when there is an infinite ground plane. When there is no infinite ground plane, a user needs to define a port as a pair of positive and negative terminals. Numerical error may be introduced if we don’t define ports as pairs without the infinite ground plane.

Select An Edge --------	There are two edge selection schemes on MGRID. One is for defining a port. A user clicks the left mouse button on the edge and the edge will be selected as a port. Another scheme is to window the two vertices of an edge.

Select A Polygon ------	In Select Polygon mode, point the cursor at a polygon and click the left mouse button.

Select A Polygon by Windowing

--------------------------	Window all the vertices of a polygon.

Side  ---------------	--------	An edge of a rectangular cell or triangular cell.

Simulation ---------------	The process of calculating circuit parameters by the simulators.

Simulation Input File --	The file created by the Set Simulation or similar process on MGRID or MODUA. It contains all the necessary information for a simulation. It contains the output s-parameter file name, the current data file name, the simulation frequencies, etc. The simulation input file will be used as the command line variable in invoking the IE3D simulation engine.

Vertex  ------------	------	A point on a polygon boundary.

Window and Windowing

---------------------------	Normally,  “window” means a rectangle on the screen. In this manual, we will also use “window” as a verb to refer to the following action series: (1) move the mouse cursor to the upper left, press down the left mouse button, drag the mouse, while the left mouse button is still being pressed down, to lower right and release the left mouse button. The action series defines the diagonal of a rectangle. The action series are referred to as windowing.

Section 2. Configuration of the Main Window of MGRID.

The basic configuration of an MGRID window is shown in Figure 2.1. It consists of a title bar, some menus, the main window in the middle, and two child windows to the right. The 3D view window is optionally opened. The 3D view window has its own menu system and we will discuss it in Section 4.

Starting from IE3D 9.0, the post-processing of current distribution display and pattern calculation is integrated into MGRID 9.0. The MGRID 9.0 also has the post-processing menu system, which will be discussed in Section 3.

The main window or the middle portion displays the top view of the circuit being edited. The child window on the upper right corner usually displays the status of MGRID. We will refer it to as status window later. In polygon input mode, the status window displays the coordinates of the mouse location (x, y, z) and the offsets to the last input vertex (dx, dy, dz, drho, dr, phi, theta). The definitions of the coordinate parameters are illustrated in Figure 2.2. The child window on the lower right corner displays the metallic layers and their corresponding z-coordinates. We will refer it as layer window. Different colors are used to identify circuits on different metallic layers. Clicking on the corresponding rectangle in the layer window usually will enable the shift of input focus to the clicked layer. The 3D view window shows the actual configuration of an actual 3D geometry.

Figure 2.1 The MGRID window.

There are 10 menus for the main window: File, Edit, Param, Input, Adv Edit, Entity, Port, Optim, View, Process and Help. Before the IE3D 9.0, there was the Element menu. The Element menu was abandoned in IE3D 9.0 for better consistency of the package. Its functionality has long been complete replaced by the MODUA simulator.

Each menu has a few menu items. A menu can be accessed though pointing the mouse cursor on the menu and clicking the left mouse button. An alternate way is to type the underlined character of the menu while holding down the Alt key. For example, you can access the File menu by typing F while holding down Alt. This action will be referenced as Alt+F in the future.  After a menu is opened, a menu item can be accessed by the mouse, or by typing in the underlined character of the menu item without holding the Alt key. If a menu item has an accelerator, you can access it using the accelerator without opening the menu first. Menu items are discussed one by one in the following. It is suggested that a user run the MGRID and try to see the location of the menus and menu items. The page numbers where the menu item is used will also be indicated in the explanation.

1. File menu.

New ------------------		Clear the memory and the windows for new circuit setup.

Open -----------------		Open a circuit geometry file (.geo) or a CURVIEW current distribution file (.cur).

Save -----------------		Save the change made to a circuit.

Save As --------------		Save the edited circuit with a new name.

Merge ----------------		Merge another geometry file into the file being edited. This feature becomes less useful after we completely implement the copy and paste command on the MGRID 9.0.

Import ----------------		Import geometry in the CIF, GDSII, 3D Text format or DXF format. DXF import/export is performed using the optional MTRAN DXF converter. For the 3D Text Format, the importing will be added into the current geometry. Other format importing will clear the memory.

Export ----------------		Export the current layout into a CIF, GDSII, 3D Text format, FIDELITY or DXF (optional) format.

Save Bitmap Picture

----------------------		Save the display as a bitmap file.

Print ------------------		Print the window.

Print Preview --------		Display the page to be printed.

Print Setup ----------		Setup printers and page orientation.

Exit --------------------	Close the MGRID window.

2. Edit menu.

Undo -----------------		Undo a modification.

Redo -----------------		Redo an undo.

2D Input -------------		Set the input mode as 2-dimensional or planar input. It requests the z-coordinate for the 2D input.

Change Layer

Z-coordinate ----		Change the z-coordinate of the current edited layer. All the vertices on the layer can optionally be moved to the new layer.

Grow Metallic Thickness

on Layer  ---------		Add the other three surfaces of the metallic strips on a 2D layer automatically. This command is useful for accurately modeling of metallic strip thickness.

3D Input -------------		Set the input mode as 3-dimensional input. Mouse input is no longer available for this mode. Vertex coordinates have to be typed in using the first two items of the Input menu. This feature is rarely used now, because the 2D Input can do a better job than 3D Input even for 3D construction. It will not be discussed in the user’s manual.

Figure 2.2 The physical meaning of the coordinates.

Select Polygon ------		Set the edit mode to selecting polygons mode. The user can use the mouse to select a polygon by clicking the left mouse button on the polygon. Selecting a polygon twice will de-select it. When this item is first selected, its focus is on all the layers. If you want to focus the selection on a specific layer, you need to click the right mouse button once or multiple times to shift the focus to a specific layer. Another way to change the selection focus is to click the left mouse button on the corresponding layer on the layer window. The information on selection focus layer is display on the status window.

To exit this mode, select Exit Edit in the Edit menu or select the Select Polygon toolbar again. The features discussed in this paragraph also apply to the Select Polygon Group and Select Vertices in the Edit menu.

Starting from MGRID 9.0, a user can get into Select Polygon mode and select a polygon directly by pressing down “Shift” and click at a polygon.

You can mix Select Polygon, Select Polygon Group and Select Vertices modes. For example, while in Select Polygon mode, you can directly go into Select Vertices mode without selecting Exit Edit first. All the vertices of the selected polygons become the selected vertices in Select Vertices mode. When you change from Select Vertices mode to Select Polygon mode, only those polygons with all vertices selected become selected in Select Polygon mode.

Select Polygon

Group -------------		Set to the mode for polygon group selection. To select a group of polygons, please window the polygons of interest. Windowing a polygon twice will de-select it. Starting from MGRID 9.0, a user can get into the Select Polygon Group mode and select a group of polygons directly by pressing down “Shift” and window the polygons simultaneously.

Select Vertices -----		Set to the mode for vertex selection. To select a group of vertices, please window the vertices of interest. Starting from MGRID 9.0, a user can get into the Select Polygon Group mode and select a group of vertices directly by pressing down “Shift” and window the polygons simultaneously. A user should understand that, when using the direct way of pressing down “shift” button and windowing a group vertices, if the windowed vertices include all vertices of one or more polygons, MGRID will enter the Select Polygon Group mode instead of Select Vertices mode.

Copy-------------------		Copy the selected polygons into the clipboard.

Paste ------------------		Paste the copied polygons from the clipboard into the current geometry.

Cut --------------------		Cut the selected polygons into the clipboard.

Delete ----------------		Delete the selected polygons or vertices.

Object Properties ---		Display the properties of the selected polygons or vertices. The user can edit the selected objects in this dialog. The metallic strip type can also be changed on this menu item if more than one metallic strip types are defined.

Change Z-coordinate

-----------		Change the z-coordinate of a group of selected vertices or polygons.

Move Objects -------		Move the selected polygons or vertices. You can move the selected objects without snapping. You can move the selected objects with snapping to a vertex. You can also move the selected objects with snapping to an edge. To switch among the different snapping modes, a user can select the Change Snapping in Edit menu (or accelerator F4).

Move at an Angle

-------------		Move the selected vertex at an angle with specified radius.

Rotate ----------------		Rotate the selected polygons around the reference point.

Copy and Reflect ---		Copy the selected polygons by reflection. MGRID will ask you the closest distance between the copied objects to the original objects.

Copy and Rotate ----		Copy the selected polygons by rotation around the reference point.

Copy at an Angle ---		Copy the selected polygons at an angle with specified radius.

Change Snapping--		Change the snapping mode in the Move Objects and Copy Objects commands.

Add Edge Vertex ---		Automatically insert vertices onto the edges of selected polygons to control meshing of the polygons.

Grow Metal Thickness

on Polygons ---------	Add the other three surfaces of a metallic strip on the selected polygons for accurate modeling of strip thickness.

Add Via on Edges ---	Build vertical polygons on selected 2D edges.

Wire Bonding -------		There are two easy ways to build a wire bond: (1) You select 2 2D edges (each edge consists of 2 vertices). Then, you select Wire Bonding to build a ribbon wire strip between the two selected 2D edges; (2) You enter two points at the two locations you want to build a wire bond. Then, you select Wire Bonding. You will have the option to build a wire bond as a ribbon or as a tube. Starting from MGRID 6.0, you can build multiple wire bonds simultaneously.

Divide Polygon -------	Divide one or more polygons into more when any entered edges are across the polygons.

Exit Edit	-------------	Exit from the Select Object, Select Object Group, Select Vertices or any other edit modes.

Escape -----------------	Escape from an editing mode such as the Paste mode, Copy and Reflect mode, etc.

3. Param menu.

Basic Parameters ----		Change the major parameters: Comment, Length Unit, Layouts and Grids, Meshing Parameters, Substrate Parameters and Metallic Strip Parameters.

Optional Parameters

---------------------	Define the optional parameters in geometry editing and simulation.

Snap To Distance Limit

--------------------		Define the range for snap to in the Copy and Move commands.

Extend -X Layout ----	Extend the layout in -X direction.

Extend +X Layout ---	Extend the layout in +X direction.

Extend -Y Layout ----	Extend the layout in -Y direction.

Extend +Y Layout ---	Extend the layout in +Y direction.

4. Input menu.

Key In Absolute Location

----------------	----	Enter an absolute location of the next vertex.

Key In Relative Location

----------------	----	Enter a vertex with offsets to the reference point. The default reference point is the last vertex. You can change it using the Set Reference Point in Input menu, or fix it using the Fix Reference Point in Input menu.

Enable Mouse Entry

---------------------		Toggle between the default drawing mode and no mouse entry mode.

Slanted Edge --------		Enter a vertex to form a slanted edge with specified length and angle.

Slanted Edge with Connection

-------------------		There are two ways in using this command: (1) You want to build an edge at a fixed angle and you want it to be connected to an edge of an existing polygon; (2) You want to build an edge with a fixed length (and approximate angle). You want the entered edge to be connected to an edge on an existing polygon.

Rotate Around Reference

---------------------	Enter a vertex by a rotation around a reference.

Orthogonal Extension

---------------------	Enter a vertex to form an edge orthogonal to the last edge.

Connect to Edge Vertically

-------------	Enable MGRID to connect the next vertex to the closest edge and make sure the entered edge is vertical to the closest edge. This feature is extremely important to make sure the geometry is exactly rectangular type. Rectangular type structure will have advantage in meshing and simulation according to efficiency and accuracy. A user can use this menu item together with the Divide Polygon in Edit menu to make arbitrary shaped polygons more rectangular shape.

Form Rectangle-------	Add the fourth vertex to form a rectangle. The condition is that there are three vertices at a right angle.

Form Polygon --------	Connect the beginning vertex and the last vertex to form a polygon.

Grid Off ---------------	Toggle the grid between on and off.

Set to Closest Vertex-	Set the next vertex to the location of the closest vertex. The default is the closest vertex on the same layer. You can type Alt+R to shift to the closest 3D vertex or the xy-coordinate of the closest 3D vertex. This state will be released only after the Set to Closest Grid is selected.

Set Parallel to Edge --	Set the entered edge parallel to an existing edge.

Set Vertical to Edge -	Set the entered edge vertical to an existing edge.

Insert Mid Point ------	Insert a vertex at the middle of an entered edge. It is not limited to the center of the edge. It can be anywhere except the two ends.

Create and Edit Vertices

--------------------		Allow a user to edit the entered vertices and import vertices from a file. Together with the Build Wire Path in Adv Edit, it is very useful for building wire antennas and complicated geometry. It is a very powerful editing feature.

Drop Last Vertex-----	Drop the last vertex of an unfinished polygon. It is equivalent to clicking the right mouse button.

Drop All Vertices-----	Drop all the vertices on an unfinished polygon.

Drop Other Than Last Vertex

---------------------		Drop some intermediate vertex.

Set Reference Point --	Set the reference point to location other than the last vertex.

Fix Reference Point --	Set the reference point to be fixed at a specified location. This item is very helpful when used together with Rotate Around Reference to draw curved structures.

Release Reference Point

------------------	Release the fixed reference point. The reference point will be the last entered vertex after you enter any vertex.

Shift Moving Reference Vertex

--------------------	This command replaces the “Set Moving Reference Vertex” in the MGRID 8.0. The “Set Moving Reference Vertex” command used to allow a user to select a vertex in the copied or moved polygons or vertices as the moving reference vertex. However, after the implementation of copy and paste in the MGRID 9.0, we have to disable such a feature because the pasted objects are from the clipboard. Instead, we implement the “Shift Moving Reference Vertex” command to shift the moving reference to the next polygon’s 1st selected vertex, to accelerate the shift of moving reference vertex.

Info on Last Entry --		Display the coordinate of the last two vertices and the total length of the entered edges. This feature is useful for measuring the dimensions of a structure.

Edit Saved Point ----		Allow you to save some dimension information into the memory for easy use in the editing. For example, you can save some offset values into the Saved Point. Then, you can get it by a mouse click when you enter a relative location using the Key In Relative Location in Input menu.

5. Adv Edit menu.

Change Dimension

Scale ----------------		Change the scale of the geometry automatically. This feature can be useful in building and sizing structures. For example, we only need to build a sphere once. Spheres of other dimensions can be created through sizing the original one. Ellipsoids can also be built by sizing the original sphere.

Flip Coordinates -----	Flip the coordinate systems. It may make geometry editing for 3D structures simpler.

Rectanglization------		Sub-divide polygons of irregular shape into rectangles. The feature will help editing and meshing of a geometry.

Check Connection ---	Check and display all the polygons electrically connected to the selected polygons.

Adjust Geometry for

Connection --------		Adjust close vertices together. If a vertex is close to an edge, MGRID will insert a matching vertex on the edge to make sure electrical connection.

Fit Selected Objects

into Grid  ---------		Adjust the x and y-coordinates of the vertices of the selected polygons to the current grid.

Align Last Vertex ---		Align the last vertex of the entered edge to be in 0 degree or 90 degree direction. For example, if you want to make an edge at 0 degree or 90 degrees, you can use Set To Closest Vertex in Input menu and enter two matching vertices to the edge. Then, you select Align Last Vertex. The location of the last vertex will be adjusted.

Align Vertices ------		Align a series of vertices on the existing polygons matching the entered vertices to be aligned with specified angles.

Align Selected Polygon Vertices

-----------------		Align the vertices of a selected polygon.

Merge Selected Vertices

-----------------		Merge the selected vertices into one location.

Merge Close Metallic Layers

-----------------		Merge the close metallic layers together.

Check Polygon

Overlapping --------	Check and display overlapped polygons.

Cut Overlapped

Polygons ----------		Cut the overlapped portions of the polygons to make sure electrical connection.

Check 3D Twisted

Polygons  ----------	Find out the 3D twisted polygons. If granted, it will break the 3D twisted polygons into multiple 3D flat polygons.

Mesh Selected

Polygons ----------		Sub-divide selected polygons into small rectangles and triangles. This feature is used to control the meshing of some polygons. For example, we want to mesh the coupled plates in an MIM capacitor in a fine scale and the other non-coupled polygons in a normal scale, we can select the coupled plates and mesh them finer first.

Merge Selected

Polygons  ---------		Merge the selected polygons into 1 (or multiple) large polygon(s).

Remove Inserted

Vertices   -----------	Remove the unnecessary vertices on selected polygons.

Dig Rectangular Hole

-------------------		Dig a rectangular hole in a 2D polygon.

Build Hole from Selected Polygons

------------------ 		Starting from IE3D 8.0, this command becomes very powerful. On one hand, it allows a user to build multiple holes of the shape of selected polygons into some other 2D polygons on layers. On the other hand, it allows user to build multiple vias through multiple 2D polygons on multiple layers. An example is shown in Figure 2.3.

Build Multilayer Vias

-------------------		This is a powerful command implemented into the MGRID 9.0 to ease the construction of multiple layer vias in PCBs.

Cut into Polygon on Edge

-------------------		Build a cut on a polygon on a selected edge.

Figure 2.3  Building multiple vias thru multiple layers using the Build Hole from Selected Polygons.

Build Path ------------	Build a 2D path with width from the entered vertices with the same z-coordinate. The lines connecting the entered vertices will be the centerline of the path.

Build Wire Path -----	Build a 3D wire path with radius from the entered vertices in the 3D space. This menu item is a more general case of the above menu item.

Continue Straight Path

----------------------	This feature is a very powerful command. It can handle multiple cases. If one edge is selected, a user is allowed to build a continuing path with different widths at the start or end. He/She is also allowed to enter a few vertices for a continuing path with multiple bends. If multiple edges on a vertical plane, such as the end of a horizontal tube, are selected, the user is allowed to continue the bend with any length.

Continue Path Bend

----------------------	This feature is also a very powerful command. It can also handle multiple cases. If one edge is selected, a user is allowed to build an abrupt bend junction. If a radius is provided, the bend will be a smooth one. The same rule applies for the cases with multiple edges on a vertical plane selected.

Build Connecting Path

---------------------	Build polygon(s) between two selected edges.

Build Connecting Paths Between 2 Layers

---------------------	Build polygons between a number of matching edge pairs between 2 layers. All the edges must be on one of the two layers. Each pair should be close in the XY coordinates.

Remove Chamfered Corners

----------------------	Replace small chamfered corners with right angle corners. Some editing tools create small chamfered corners. These small chamfered corners will have little effect on a circuit’s performance. However, we would like to replace them with right angle corners in consideration of numerical efficiency.

Remove Redundant Vertices on Curvature

----------------------	Some imported geometry has many vertices on curvatures. The vertices will force the meshing program to create many redundant cells. The redundant cells will slow down the simulation and do not improve the accuracy. In fact, the long and tiny redundant cells may create numerical singularities in simulation. You can use this menu item to reduce the number of vertices on the curvature.

Convert Polygons in Shape

----------------------	This command allows a user to select a group of polygons and convert the polygons into rectangles or circles with default size for each polygon or some specified size.

6. Entity menu.

Enclosure Walls ------	Allow a user to define vertical electric and magnetic walls in the structure. You need to specify the number of images used in the simulation. The specified number will be the maximum number of images used. The actual image number will be used if it is smaller than the specified number. It is useful for modeling the enclosure walls and symmetrical structures.

Probe-Feed to Patch

----------------------		This command allows a user to build a probe with a vertical localized port defined on it. It is extremely useful in modeling probe-fed patch antennas and wire antennas.

Conical Vias ---------		This command allows a user to build one more conical vias between two layers automatically.

Other menu items in Entity menu are for one-step construction of complicated structures such as circular patches, helix antennas, cylindrical tubes and spiral structures. They will be discussed in Appendix I.

7. Port menu.

Define Port ----------		Set to define port. The user can use the mouse to select one or several sides or edge of a polygon as a port. Usually, left mouse is for the positive ports and right mouse is for the negative ports. For each port, more than one isolated negative port can be defined. MGRID will not resume previous edit mode until the user select Exit Port in the Port menu.

Port for Edge Group

--------------------		Set to define a group of edges for a port. The user can use the mouse to define a rectangle to enclose open edges for a port. Other features of the last item apply to this item.

Selected Rectangles for Horizontal Localized Port

--------------------		The only way to define a horizontal localized port, a port style implemented in IE3D 6.X. To define a horizontal localized port, a user should select a group of horizontal rectangles. Then, he/she selects this menu item. MGRID will perform the checking and define the selected rectangles as a horizontal port.

Selected Edge for Extension Ports

--------------------		The alternative way to define an extension port. On Define Port or Port For Edge Group commands, a user selects the edges after selecting the command. For this command, the user selects the edges first. Then, select the menu item to define an extension port.

Selected Rectangles for Vertical Localized Port

-------------------		The alternative way to define a vertical localized port.

Ground Port Conversion

----------------------		Convert all the individual negative ports into a common negative port that is the ground port.

Multiple Positive Port

---------------------		Selected to define multiple positive ports with the same port index. After you define a positive port, you can select this menu item. Then, the next defined port will not have the next port index. It will have the same port index as the one defined before. For example, we just define the positive port 1. If we do not select Multiple Positive Port in Port menu, the next port defined will be the positive port 2. If we select the Multiple Positive Port in Port menu immediately after we define the positive port 1, the next port will also be the positive port 1.

Define Reference Plane

----------------------		Immediately after each port (including positive and negative ports) is defined, a user can define the reference plane shift. The default reference plane will be where you define the port. You can use this feature to define the reference plane to location other than the default reference plane. Reference plane shift for coupled ports may introduce error. Please use the back simulation concept in MODUA to perform reference plane shift for coupled port structures.

Drop Last Port ------		Drop the last defined port. This item is useful for port definition correction.

Change Scheme ------	Change the port parameters.

Change Port Order ---	Change the orders and signs of the ports.

Delete Port ------------	Set the edit mode to delete port. The user can use the mouse to select a port to be deleted. MGRID automatically resumes the previous mode after a port is deleted.

Delete All Ports ------	Delete all the ports.

Delete Port Range ---	Delete a number of ports.

Plane Wave Excitation

------------------		Define the incident angles of a plane-wave excitation.

Remove Plane Wave

---------------------		Remove the plane-wave excitation.

Exit Port -------------		Exit the state of Define Port, etc.

8. Optim menu.

Variable For Selected Objects

----------------------		Define the selected vertices as an optimization variable. The user will be prompted for the direction and bounds for the variable. This command and the next command are interchangeable in MGRID 9.0.

Add Selected Objects to Variable

------------------------		Add the selected vertices as a call to an existing optimization variable. The user will be prompted for the change direction and tuning rate. This menu item is useful to define a variable to change different vertices at different directions and/or rates.

Change Variables and Calls

----------------------		All the users to access all the defined variables and calls. This command removes the Change Variable Bounds, Remove Variables and Fix Vertices commands in the MGRD 8.0 version.

Display Trends -------	Display the low bound and high bound of optimization variables.

Exit Optim ------------	Exit any unfinished process in menu Optim.

9. View menu.

Toolbar ---------------	Open or close the toolbar.

Status Bar -------------	Open or close the status bar.

3D View ---------------	Open or close the 3D view window.

Zoom 25% to Zoom 400%

----------------------	Zoom the display 25% to 400%.

Whole Circuit -------		Display the whole circuit on the window. For those versions before Version 4.0, MGRID will create a different grid for the whole circuit view. Starting from Version 4.0, the grid will be preserved. The grid system can only be changed in the Basic Parameters dialog in the Param menu.

Zoom -----------------	Zoom portion of a circuit by windowing it.

Element Connected View

-----------------------		By default, it is unchecked and all the ports of the structure are displayed. When checked, the ports connected to lumped elements will not be displayed.

Show Element --------	Toggle between showing and not showing lumped elements.

Show Metal Type ----	Toggle between showing and not showing metal type of each polygon.

Show Disconnected Vertices

----------------------		Toggle between showing and not showing disconnected vertices. Any vertex of a polygon not common to another polygon is considered as a disconnected vertex.

Show Ground Connection

----------------------		Toggle between showing and not showing polygon connection to ground. By default, any 3D polygon with an edge connected to the ground interface is considered grounded. However, it is difficult to tell from the top view. When checked, the ground connection will be shown with a thick red line.

Show Port Extension

---------------------		Toggle between showing and not showing the port extensions. By default, the port extensions are displayed with gray lines. Users should try avoiding the overlapping of the port extensions with the structure for accurate modeling.

Keep Discretized

----------------------		Toggle between keeping and not keeping the geometry discretized. By default, the discretized circuit is displayed once every time you mesh it. You can check this item to keep it until your circuit is modified.

All Layers -------------	Displaying the objects on all metal layers simultaneously.

Single Layer ---------		Displaying the objects on a specific layer only.

Elevate Layer--------		Displaying the objects on a specific layer on top of other players. The structure is not changed. Only the view is changed.

10. Process menu.

Display Meshing ----		Perform the meshing of the geometry into small rectangular and triangular cells and displays the result on the window. This result is only for checking only. It is not necessary for a simulation. The circuit will be meshed again in the simulation engine when it is being simulated. The final mesh may be different due to different options.

Simulate ---------------	Setup an electromagnetic simulation of the current geometry.

Optimize --------------	 	Setup an electromagnetic optimization on the structure. Different optimization schemes are available. (1) Powell Optimizer; (2) Genetic Optimizer; (3) Random Optimizer. The Powell Optimizer is the local optimizer. It is good for simple goals. The Genetic Optimizer and Random Optimizer are global optimizers. They are more robust. However, they normally are slower.

A user should understand that optimization is a mathematically unsolved problem. For a general optimization problem, you even do not know whether you will have an answer or how many possible results you will have. Before you start an optimization, you need to make sure you will have an answer, and error functions are sensitive to the variables you defined. You also need to define the right bounds. Wider bounds certainly can more likely avoid going out of the bounds. However, wider bounds may give the optimizer too much freedom to go to other location.

Display S-Parameters

----------------------		Invoke MODUA for s-parameter display.

Display Radiation Pattern

----------------------		Invoke PATTERNVIEW for pattern display.

11. Help menu.

Help Topics -----------	Starting from IE3D 6.0, we offer the help on the menus.

About MGRID -------	Display the version number of the MGRID.

Besides the menu items, there are some bitmap pictures in the toolbar. Each bitmap corresponds to a frequently used command, which may correspond to a menu item. The toolbar is located between the menus and the main window.

Section 3. The Menu System of MGRID as the Post-Processor.

Current distribution display and radiation pattern calculation used to be on the CURVIEW applications. On the IE3D 9.0, the current distribution display and pattern calculation features are improved significantly. They are also integrated into the MGRID for better integrity. The CURVIEW will be phased out after version 9.0. To use the MGRID as the post-processor, please open the CURVIEW (.cur) file created in an IE3D simulation. The MGRID will automatically switch to the post-processor menu system (see Figure 2.4).

Figure 2.4  The window configuration of MGRID as post-processor.

1. File menu.

New ------------------		Clear the memory and the windows for new circuit.

Open -----------------		Open a circuit geometry file  (.geo) or a CURVIEW current distribution file (.cur).

Save Current Density Data

---------------------		Save the current distribution data with the specified excitations and terminations into a file.

Print ------------------		Print the window.

Print Preview --------		Display the page to be printed.

Print Setup ----------		Setup printers and page orientation.

2. Process menu.

Display Structure ----		Clear the memory and the windows for new circuit.

Display Current Distribution

----------------------		Open a circuit geometry file  (.geo) or a CURVIEW current distribution file (.cur).

Pattern Calculation

-----------------------		Start a pattern simulation with generalized excitations and terminations. Then, a user can save the pattern with specified excitations and terminations. Future version will include excitations and terminations optimization for patterns.

Display S-Parameters

----------------------		Invoke MODUA for s-parameter display.

Display Radiation Pattern

----------------------		Invoke PATTERNVIEW for pattern display.

3. View menu.

The View menu consists of parts of the menu items in the View menu of the main MGRID window menu system.

The post-processor window of MGRID allows a user to select polygons. A user can choose to calculate the radiation patterns of the selected polygons or the whole structure.

Section 4.  The 3D View of MGRID.

The 3D view of MGIRD is implemented based upon the OpenGL. The View menu in the 3D view window allows a user to change the angles of the display.

When MGRID is in editing mode, a user can also choose to display markers on the connections between polygons. To rotate the 3D view, a user can also press down the left mouse button and move the mouse. To zoom the 3D view window, a user can press down “Ctrl” and window the portion for zoom. To pan the view, a user can press down the right mouse button and move.

When MGRID is in post-processing mode, a user can choose the Save Bitmap Files button to save the animation pictures into bitmap files. The saved bitmap files can be used by ZDibAnimator for animation again. The animation on MGRID does not create any bitmap files. However, the animation on ZDibAnimator is based upon saved bitmap files. In case, a user creates a meaning animation and he wants to play later, he should save the bitmap files and play it again on the ZDibAnimator.

Section 5.  Configuration of the MODUA Window.

As we have mentioned, MODUA is basically a nodal circuit simulator. It also serves as the graphic display tool for the IE3D simulation result. It accepts circuit elements as black boxes, which are called modules.

There are 10 module types in MODUA: geometry file, parameter file, resistor, capacitor, inductor, mutual inductor, port, connection, short circuit and open circuit. The 10 module types are sub-divided into two groups: independent modules and dependent modules. Geometry files, parameter files, resistor, capacitor and inductor are independent modules. They can be defined anywhere at any time. Port, mutual inductor, connection, short circuit and open circuit are dependent modules. At the time the dependent modules are defined, they must be connected to the terminals of the independent modules. When an independent module is deleted, the dependent modules connected to this independent module will also be deleted.

In some sense, MODUA is a circuit simulator similar to the Touchstone from HP/EEsof. However, MODUA does not have a big library. Because it is basically implemented for the post processing for IE3D, it really does not need a big library. It accepts geometry modules from MGRID. Therefore, you can build any structure elements on MGRID and import them into MODUA.

The basic configuration of an MODUA window is shown in Figure 2.5. It consists of seven menu items: File, Edit, Element, Control, Process, View and Help.

Figure 2.5 The MODUA window.

1. File menu.

New -----------------------	Clear the memory and the window for a new design.

Open ----------------------	Open a design.

Save -----------------------	Save the change to the current design.

Save As ------------------	Save the current	 design with a new name.

Add Geometry Module

------------------------	Add a geometry file as a module of the design.

Add Parameter Module

------------------------	Add a parameter file as a module of the design.

Display Parameter Module

-------------------------	Fast setup of the MODUA design for display of parameter file.

Parameter File Queue

-------------------------	Add parameter files into the queue for simultaneous display and comparison of different s-parameters.

Save S-Parameters into Queue

-------------------------	Save the current s-parameters into a file and put it into queue automatically.

Save S-Parameters ------	Save the current s-parameters into a file.

Save Displayed Data ---	Save the displayed s, y, z-parameter data list into a text file.

Open Matrix File

--------------------------	Open a file containing the RLC matrix elements (an IE3D specific format). For transmission line structure, RLC topology is in matrix forms. It is easier to manipulate the matrix than the SPICE netlist. A user can use the IE3D to model a small portion of a package structure and extract the RLC matrices. They can use the local RLC matrices to expand it into an overall RLC matrix using a spreadsheet application. Then, he can import the RLC matrices back to the IE3D and convert it into SPICE file.

Save Matrix File

--------------------	Save the transmission line RLC equivalent circuit into an IE3D specific matrix file.

Save SPICE File

--------------------	Save the transmission line RLC equivalent circuit into a SPICE netlist file.

Import SPICE File

--------------------	Import a SPICE netlist file (containing R, L, C and K only) into the current design. This feature is very important on checking whether an equivalent circuit is working fine. For example, you use the RLC equivalent circuit feature in IE3D to extract the SPICE circuit. You do not know how good the equivalent circuit is. You can import it back to MODUA and do a simulation. Then, you can compare the simulation result of the equivalent circuit and the original circuit to find out what is the valid frequency range for the equivalent circuit. You can even fine-tune the SPICE model using the Match Queue in Process menu.

Export SPICE File

--------------------	Export the current design (R, L, C, mutual inductor and short circuit only) into a SPICE netlist file.

Merge Matrix Files ---	This is a useful command for modeling large signal integrity (SI) structure. For example, we may have a lead frame with tens or hundreds of leads. If we break it into smaller size to simulate, the result may not be accurate for some leads on the edge of the size. The Merge Matrix File allows a user to merge the extracted RLC circuits of the smaller sections in matrix form to get the complex RLC circuit in matrix form for the whole structure.

Save Excitation ------	Save the result of the Simulate and Find Excitation in Process menu. The saved file can be imported into CURVIEW. It is critical to finding the current and radiation patterns of structures with lumped elements or antenna array.

Save TLN Information

--------------------	Save the transmission line parameters into a text file.

Print ------------------	Print the window.

Print Preview -------	Preview the print out of the window.

Print Setup ----------	Setup the printer for printing.

Exit -------------------	Close the MODUA window.

2. Edit menu.

Select Module ----------	Set the input mode to select module mode. In select module mode, you can move the module by dragging it. You can use other menu items to control the modules. It is the default mode in MODUA now.

Module Properties -----	Display the module properties of the selected module.

Rotate Module  --------	Rotate the direction of the module or the ports of the module.

Fix Module ---------------	Fix the location of the modules in the current location.

Delete Module ---------	Delete the selected modules.

Replace Module -------	Replace the selected module with another module.

Set Selected RLCM modules for Optim

---------------------------	Set the values of the selected R, L, C and mutual inductor as optimization variables.

Remove Optim from Selected RLCM

---------------------------	Set the selected R, L, C and mutual inductor to be fixed value components.

Delete Dependent Modules

---------------------------	Delete all dependent modules.

Set all RLCM as Optimization Variables

---------------------------	Set the values of all the R, L, C and mutual inductor as optimization variables.

Remove RLCM Optimization Variables

---------------------------	Remove all RLCM as optimization variables to their initial values.

Exit Status ----------------	Exit the select module mode.

3. Element menu.

Port -------------------------	Generate a port module. The Port module needs to connect to a terminal of an independent module.

Define All Ports---------	Define all the spare terminals of the modules as ports of the design.

Connection --------------	Generate a connection module. You need to connect the two terminals of a Connection module to any two terminals of independent module(s). When you use the Connection module to connect the two terminals of any independent module(s), the two terminals will be of equal potential.

Short Circuit -------------	Generate an idealized short circuit. The Short module needs to connect to a terminal of an independent module.

Open Circuit -------------	Generate an idealized open circuit. The Open module needs to connect to a terminal of an independent module.

Termination -------------	Allow a user to build a resistor with one terminal short-circuited.

Resistor -------------------	Generate an idealized resistor. You can define the value of the resistor as an optimization variable.

Capacitor ----------------	Generate an idealized capacitor. You can define the value of the capacitor as an optimization variable.

Inductor -------------------	Generate an idealized inductor. You can define the value of the inductor as an optimization variable.

Mutual Inductor --------	Generate a mutual inductor. You can define the value of the mutual inductor as an optimization variable. A Mutual Inductor module is a dependent module. It must be defined between two inductors. Each pair of inductor can have only one single Mutual Inductor module. The Mutual Inductor takes value range in (-1,1). You can control the sign of the mutual inductor using the value or the connection. Connecting the mutual inductor at the port 1 of each inductor is the same as connecting the mutual inductor at the port 2 of each inductor. When you connect the mutual inductor at the port 1 of an inductor and port 2 of the other inductor, it means that the value of the mutual inductor will have a sign change in the calculation.

Exit Element -----------	Exit from the element status.

4. Control menu.

Frequencies ------------	Define the frequencies of simulation.

Define Display Data --	Define the parameter items for list display.

Define Display Graph-	Define the parameter items for graph display.

Define Display Smith Chart

------------------------	Define the parameter items for the Smith Chart display.

Terminating Impedance

-------------------------	Define the termination or normalization impedances of the ports.

Change Excitation ------	In displaying the result of the Simulate and Find Excitation in Process menu, you can change the excitation and the loads of the ports. This feature is good for finding out the excitation and loading of antenna arrays or antennas with lumped elements.

Display Toggle ----------	Toggle between design module display and parameter display.

5. Process menu.

Simulate ----------------	Perform a simulation on the design.

Simulate and Find Excitation

-------------------	Perform a simulation on the design. After the simulation, it will display the power, voltage, current and load at each port. A user can then select Change Excitation in Control menu to adjustment of the excitation and loading. The result can be saved into a file by selecting the Save Excitation in File menu. The saved excitation file can then be imported into CURVIEW for displaying current and radiation patterns of structures with lumped elements or antenna arrays.

Batch Simulate and Find Excitation

-----------------------	Allow a set of simulations on a design with an s-parameter file replaced for each simulation.

Optimize ----------------	Perform an optimization on the design. Please refer to the Set Optimization menu item of the MGRID for more information.

L - Equivalent  ------	Perform simulation and solve for the L-matrix of a design. L-equivalent is good for structure with dominant inductive effect. It is not as accurate as the LC-equivalent in discussed next. However, it reduces the number of elements.

C - Equivalent ------	Perform simulation and solve for the C-matrix of a design. C-equivalent is good for structures with dominant capacitive effect. It is not as accurate as the LC-equivalent in discussed next. However, it reduces the number of elements.

LC - Equivalent -----	Perform simulation and solve for the LC-matrix of a design. This is the best equivalent for transmission line.

Match Queue File ----	This feature is useful in solving the RLC equivalent circuit of an arbitrary design. The above 3 menu items are only for coupled transmission line models. The model always assumes shunt C and R to ground and series L and R between input and output. They cannot be applied to any other structure such as series L and C. It is not realistic to build in different models for different structures. The Match Queue File feature will solve the problem. Basically, it allows a user to construct any kind of the RLCM topology on MODUA. MODUA will try to match the values of the RLCM into the s-parameter file in the Parameter File Queue. For example, you have a complicated structure that cannot be fitted into the transmission line model. You simulate the s-parameter and save the result into a file. You put the file into Parameter File Queue. Then, you start building the RLCM model on MODUA. You set the appropriate RLCM modules as optimization variables. You define the appropriate frequency points and select the Match Queue File menu item. MODUA will try to fine tune the optimization variables and match the performance of the design to the parameter file in the queue. If it is successful, the RLCM topology will be a good SPICE representation to the parameter file in the queue. Sometimes, it may take a few runs before you can get a good result.

The Match Queue File is also good for fine-tuning the LC-equivalent circuits. A user should understand that the LC-equivalent circuit is extracted at one single frequency point only. It works in a frequency range around where the extraction is performed. If you want to make it working in a wide frequency range, you can fine-tune the values of the LC-equivalent circuit based upon the s-parameter results. You may even need to change the LC-equivalent circuit and try to match the s-parameters in the queue for better result.

Back Simulation ------	This feature makes the IE3D de-embedding even more flexible. One of its most important applications is in shifting reference plane for an IE3D simulation. Shift reference plane is available on MGRID for isolated port. Whenever you encounter a shift of reference plane, the IE3D will simulate a short uniform line to calculate the transmission line parameters that will be used to do the shift reference. There are cases the shifting reference plane feature on MGRID may not work well. One case is that the ports are closely coupled together. When a few ports are coupled together, transmission line parameters are dependent on the excitation of the port parameters. Another case causing problem is a long distance reference plane shift. For example when we shift the reference plane by one wavelength, we need to change the phase of S21 by 720 degrees. If we introduce 0.1% error in calculating the wavelength, we will introduce 0.72-degree error in the phase of S21. If we shift the reference plane 10 wavelengths, the error in the phase of S21 may be 7.2 degrees. The problem in the magnitude of S21 is even more serious. This kind of error is actually understandable. However, a user will not be happy if he simulates a uniform transmission line of 10 wavelength long and shifts the reference planes to the center of the line and he gets the phase of S21 to be 7.2 degrees. He may demand higher accuracy. The shift reference plane feature in IE3D cannot provide much higher accuracy. He can get higher accuracy result by doing the following. He/she simulates the 10-wavelength long transmission line and a 5-wavelength long transmission line. By using the Back Simulation feature on MODUA, he can remove the 5 wavelength long transmission line on each port of the 10 wavelength long transmission line to get the s-parameter reference at the center of the line and the accuracy will be much higher than the shift reference plane feature in MGRID. In fact, the benefit of the Back Simulation goes beyond the shift of reference plane of uniform transmission lines. You can even use it to shift the reference plane of non-uniform transmission lines.

Separate S-Parameters

-----------------	This is a very powerful feature. It allows a user to separate an s-parameter file into two connected s-parameter files. It is very similar to the Back Simulation feature. However, this is a more general feature.

Remove S-Parameters

----------------	This is also a very powerful feature. It allows a user to remove the effect of coupled transmission lines on the ports. It is a command can replacing the Back Simulation. The Back Simulation can only remove the effect of isolated transmission lines on the ports.

Find Transmission Line Parameters

---------------	Solve for the characteristic impedance and other transmission line parameters of the s-parameter file of a structure. The structure must be a uniform 2-port structure shorter than quarter wavelength. When the structure is a uniform microstrip or stripline structure, the IE3D simulation engine will automatically detect it and solve the parameters as save them into the s-parameter file as comment line. For some complicated uniform structure such as CPW with finite ground plane or coupled microstrip line, we defined positive port and negative port and the IE3D simulation engine cannot detect it is a 2-port uniform structure. It will not solve for the transmission line parameters automatically. Then, you can use this feature in MODUA to find the parameters. It will prompt you for the length of the transmission line in mm. The characteristic impedance will be correctly calculated no matter what kind of number you enter it as the length. Entering a value other than the actual one certainly will affect the propagation constant and effective dielectric constant.

Create TLN S-Parameters

----------------------	Allow a user to create an s-parameter file for the frequency response of an ideal transmission line based upon the electrical length, characteristic impedance and transmission parameters.

Curve Fitting and Interpolation

-----------------	Curve-fitting the s-parameters based upon the existing frequency points.

Create S-Parameter File

---------------	Create a frequency independent s-parameter file.

Re-Normalize and Save

---------------	Allow the user to re-normalize the s-parameters and save it.

Find Perfect Load ------	Given a 2-port s-parameter file over a frequency range, the feature allows a user to find the 1-port s-parameter frequency response to be connected to the port 2 of the 2-port s-parameter file for a perfect match at port 1 of the 2-port s-parameter file. It can be a useful tool for wide band antenna design.

Differential Conversion

----------------------	This feature allows a user to convert an existing s-parameter file into a new s-parameter file with some of the ports differentially excited.

Odd and Even Mode Conversion

----------------------	This feature allows a user to convert the s-parameters of uniform coupled TLN into the s-parameters based upon odd and even modes.

6. View menu.

Toolbar ------------------	Open or close the toolbar.

Status Bar ---------------	Open or close the status bar.

Design Identification--	For the identification of the simulation result

Design View Toggle -----	Toggle between displaying the module ID and displaying the values of the RLCM modules.

Graph Parameters ----	For change of graph parameters such as the range of the parameters, digits of the text, and the graph sizes.

Display Grid Toggle ---	Toggle between displaying the grid and displaying no grid.

Optional View Settings

-----------------------	Allow the user to define the settings. For example, you can setup MODUA’s default display mode. The initial default display mode for MODUA is Smith Chart display. You can change it to some other mode.

Extend Horizontal Scroll Bar

--------------------------	Extend the horizontal viewable size.

Extend Vertical Scroll Bar

-------------------------	Extend the vertical viewable size.

Display Queue Items----	Defining the queue parameter items to be displayed and compared.

7. Help menu.

About ----------------------	Display the version number of the MODUA.

No on-line help is available at the present time.

Section 6. Configuration of the IE3DLIBRARY Window.

The IE3DLIBRARY is introduced in the IE3D 9.0 as the 2nd interface of the IE3D. Before the IE3D 9.0, MGRID was the only layout editor for geometry construction. MGRID has been a very powerful graphic interface. MGRID is based upon low level objects: polygons and vertices as. We can create and modify polygons and vertices, which are the basic elements of a structure, to achieve the shapes of a circuit. Users can also create high-level objects such as circular spiral inductors, MIM capacitors with grounding via on MGRID. However, after the high-level objects are created, they will be broken down into polygons, the basic objects. The high-level objects cannot be edited as an entity on the MGRID. It would be nice if we can create and edit a high-level object as an entity. Implementing such a scheme on the MGRID is quite tough because of the way MGRID is handling the polygons and vertices. For this reason, we have implemented the IE3DLIBRARY. The IE3DLIBRARY allows a user to create a circuit or antenna based upon high-level objects. The high-level objects are described as a set of polygons. However, the user does not have direct access to the polygons. Any further editing has to be done on the objects. Therefore, the integrity of each object is maintained. Compared to MGRID, the IE3DLIBRARY is much easier to use to create some commonly used structures. However, the IE3DLIBRARY cannot access the small details of a structure. It would not be as flexible as the MGRID for general structures. Any structure created using IE3DLIBRARY can be exported into the MGRID. Also, the user should be notified that the IE3DLIBRARY is still in its infancy. It still needs much effort to make it equally powerful as the MGRID.

The typical IE3DLIBRARY window is shown in Figure 2.6. There are 5-menus items in it. On the File menu, a user can start a new file, open or save a file. The file extension is .ie3. Also, a user can save the file as .geo file and invoke the MGRID will open the saved .geo file for further processing. The Edit menu has the basic select, copy, cut and paste commands. There are extra commands we will discuss later with detail. All the objects are in the Insert and Module menus. A user can select any menu item in the two menus to define an object, and then place and connect it to the objects in the layout. On the Edit menu of the IE3DLIBRARY, there is a menu called “Tuning Variables…” A user can define a set of variables. Then, he can associate some dimensions in the objects with the variables. After we build the structure with electrical connection between objects, we can still change the shape of the objects using the tuning variables. When we change the tuning variables, the dimensions associated with the variables will change correspondingly. However, the electrical connections between the objects will be kept. Such a feature makes it simple to modify and tune a circuit. Eventually, we will build an optimizer on it so that we can tune the parameters of a circuit easily.

Figure 2.6  The IE3DLIBRARY window with 3 objects on it.

Section 7.  Configuration of the CURVIEW Window.

CURVIEW has been the post processor of the IE3D to display current distribution, radiation pattern and near field distribution. Starting from the IE3D 9.0, its major functionality has been improved and integrated into the MGRID. The CURVIEW should be phased out some time after the IE3D 9.0.

The basic configuration of a CURVIEW window is shown in Figure 2.6. It consists of six menu items, File, Process, View, Current Display, Field Display and Help.

1. File menu.

Open --------------------	Open a current distribution data file created by IE3D.

Open Field Data File--	Open a field data file saved before. You can only open the field data file after you open the current distribution data file from which the field data file was created. It also requires you select the right frequency point.

Save Field Data---------	Save the field data created by the Field Plot in Process menu. The near field calculation takes a long time. You can save it into a file for later use.

Open Radiation Pattern Data File

----------------------------	Open a saved radiation pattern data file for displaying radiation pattern. The saved radiation pattern data file can only be opened after you open the associated current distribution data file at a specific frequency. This feature only allows opening the pattern files in version 5.X or earlier. Only the Save Radiation Pattern Data command can save the pattern into version 5.X format. The more powerful command in saving pattern data is the Radiation Pattern Vs. Frequency in Process menu. It will save the pattern data in version 6.X or later format. Only the PatternView application can open the pattern data in version 6.X.

Figure 2.7 The CURVIEW window.

Save Radiation Pattern Data

----------------------------	The command to save the CURVIEW displayed pattern into pattern file format 5.0. This feature is being replaced by the Radiation Pattern Vs. Frequency in Process menu.

Save Current Data Text File

----------------------------	This current data file is not in the same format as the one created by IE3D. This file contains information on the vector current density at a specific point. It cannot be read by CURVIEW.

Save Bitmap File--------	When the CURVIEW windows is displaying a bitmap-based picture, selecting this menu item will save the picture into a bitmap file. You can use other Windows programs to open the file for further process.

Print -----------------------	Print the window.

Print Preview -----------	Preview the print out.

Print Setup --------------	Setup the printer.

Exit -------------------	Close the CURVIEW window.

2. Process menu.

Structure View----------	Show the 3D structure in the current distribution data file.

2D Vector Current Density Plot

--------------------------	Display the vector current at a specific time. You can change the time by selecting the Change Phase in Current Display menu.

3D Current Density Animation

---------------------------	Animation of sinusoidal current distribution on a structure.

3D Average Current Density

---------------------------	Display the current density magnitude at different location of a structure.

Start Field Calculation

----------------------------	Start a calculation for near field.

Display Radiation Pattern

---------------------------	Start a calculation for the radiation pattern.

Radiation Pattern Vs. Frequency

---------------------------	This is new on IE3D 5.0. It allows you to create the radiation patterns at a series of frequencies. Then, you can use the PatternView to display and compare the patterns at different frequencies and from different files.

Select Cells ------------	Select the cells you want to calculate the radiation.

3. View menu.

Toolbar -----------------	Open or close the tool bar.

Status Bar --------------	Open or close the status bar.

Original View----------	Get back to the original view.

Zoom---------------------	Change the zoom factor. A user can also zoom a portion of the display by defining a rectangle enclosing the portion of interest.

Toggle Electric and

Magnetic  -------------	Toggle displaying electric current distribution and magnetic current (slot electric field).

Change Angles--------	Change 3D display angles.

Increase Theta --------	Increase the elevation angle by 5 degrees. The elevation angle is defined in Figure 2.2.

Decrease Theta ------- 	Decrease the elevation angle by 5 degrees.

Increase Phi ------------	Increase the azimuth angle by 5 degrees. The azimuth angle is defined in Figure 2.2.

Decrease Phi ----------- 	Decrease the azimuth angle by 5 degrees.

Display Option ---------	Change display options here. You can change the scale of the color bar, control display color using bitmap or polygons, etc.

Display Layer------------	Display a specific layer by specifying the layer index. Display all layers except the specific layer by specifying the negative value of the layer index. Display all layers by specifying 0.

Display Coordinate Frame

-------------------------	Choose between displaying and not displaying the coordinate frame.

Rotate Coordinate System

------------------------	Rotate the coordinate system by 90 degrees. After the rotation, the definition of the azimuth angle (j, or phi) and elevation angle (q, or theta) are different. For the default (x, y, z) coordinate system, q and j are defined in Figure 2.2. For the (y, z, x) coordinate system, you need to replace x with y, y with z, and z with x. For the (z, x, y) coordinate system, you need to replace x with z, y with x and z with y.

Port Information--------	Display voltage/current, or incident/reflected waves at each port.

Display Title ------------	Allow a user to enter a title for the display.

Layout Editor Comment

------------------------	You can display the layout editor comment here. The layout editor comment will be printed on the print out.

4. Current Display menu.

Change Frequency ---	Change the frequency of the displayed current distribution.

Change Excitation ----	Change the excitation of each port. You can import the excitation file created by MODUA. The importing excitation files are necessary for displaying current and radiation patterns of structures with lumped elements or antenna array.

Change Vector Size---	Change the vector size of 2D Current Display.

Change Timer Param-	Change the timer parameters for current animation.

5. Field Display menu.

Field Display Cut Planes

----------------------------	Define the cut planes for near field display.

Location Label----------	Define label display interval.

2D Radiation Pattern--	Define 2D radiation pattern display items.

2D Polar Pattern --------	Define 2D polar pattern display items.

3D Pattern ----------------	Define 3D radiation pattern display.

3D Mapped Pattern---	Define 3D mapped radiation pattern display. The difference between the 3D mapped radiation pattern and the 3D radiation pattern is similar to the difference between 2D radiation pattern and 2D polar radiation pattern. Basically, 2D radiation pattern is the cut view of 3D mapped radiation pattern and 2D polar pattern is the cut view of 3D radiation pattern.

Radiation Parameters-	Define displaying radiation pattern parameters such as directivity, efficiency, monostatic RCS or bistatic RCS.

Array Factor--------------	Define an array factor to apply to the element radiation pattern.

6. Help menu.

About ----------------------	Display the version number of the CURVIEW.

Now that all the menu items have been explained, we will go ahead to try an example. Whenever a menu item is unclear to you, you can go back to this chapter for a reference.

Most of the simulations documented in this manual are performed on a Pentium III-M, 1.13 GHz notebook computers with 512 M RAM. Because the IE3D is a computational intensive program, the simulation speed is almost independent of the RAM amount when it is not swapping. Its speed is almost proportional to the clock-rate of the CPUs for the same class of CPUs. Table 2.2 lists the speed comparison. The IE3D 6.2 and later has an implementation of the SMSi matrix solver, which is the default matrix solver. The SMSi is about 4 times of the speed of the SMS matrix solver. The SMSi matrix solver is available on any Win32 machines with Intel Pentium Pro, Pentium II, Pentium III, Pentium 4 and AMD Athlon CPUs. For multiple CPU systems, there is no speed improvement for 1-single simulation. However, if you are running multiple simulations, the multiple CPU systems will be significantly fast. We expect 2 simulations running on a 2-CPU system will be about the speed of 1-simulation running on a 1-CPU system. Starting from the IE3D 8.2, the Zeland Distributed Computer Service (ZDS) is released for network based distributed computing. Interested users can read the Appendix and contact Zeland Software, Inc. for the licensing.

One of the major breakthroughs in the IE3D 8.0 is the AIMS iterative matrix solver. The AIMS solvers allow the IE3D to solve extremely large planar structures such as large arrays efficiently using little RAM. The applications of AIMS will be discussed in the Chapter 13.

Some users may be curious about the speed comparison between the Intel platform and other RISC platforms. A complete comparison is not available. However, we have performed a comparison between the IE3D on Windows NT for Intel platform and the IE3D on Windows NT for DEC-Alpha platform. Using the SMS matrix solver, the IE3D on a 500-MHz DEC-Alpha 21164 CPU is about 90-130% of the speed of the IE3D on 450-MHz Pentium II CPU in 1998. With the same clock-rate, we do not see the speed advantage of the DEC-Alpha 21164 over the Intel Pentium II CPU. The implementation of the SMSi matrix solver on the IE3D for Windows NT for Intel changes the situation much. For a typical 1500 unknown problem, the IE3D using SMSi is about 3 times as fast as the IE3D using SMS matrix solver. The SMSi matrix solver is only available on the IE3D for Windows NT for Intel. In the other words, the IE3D for Windows NT on Intel can be much faster than the IE3D on other platforms of the same clock-rate. In fact, the IE3d on other RISC machines are no longer supported in the latest releases.

All the files described in this manual will be saved in the c:\program files\zeland\ie3d\samples directory. To simplify the file name, we will assume the directory is c:\ie3d\samples. It is suggested that users do not change them. Users should save their practice files into c:\ie3d\practice or their own directories. The benchmark example files are saved in c:\ie3d\examples directory. The IE3D executables are located in c:\program files\zeland\exe directory.

Table 2.2  The speed comparison for the IE3D 9.0 on different platforms

| CPU | 486DX2-66 | P-90 | P-200 | Pentium-Pro 200 | P-II 400 |
| --- | --- | --- | --- | --- | --- |
| Clock-Rate | 66 MHz | 90 MHz | 200 MHz | 200 MHz | 400 MHz |
| Speed | 1 | 2 | 4.5 | 6-9 | 12-18 |
| CPU | P-III 800 | Athlon | P-4 2.2 G |  |  |
| Clock Rate | 800 MHz | 800 MHz | 2.2 GHz |  |  |
| Speed | 24-36 | 36-54 | 30-180 |  |  |